ELECTRICAL AND ELECTRONICS ENGINEERING

OPTION:

- ELECTRONICS AND TELECOMMUNICATION TECHNOLOGY

REQF level:6                                  Class: Y1

Module Name: Microprocessor and Microcontroller

Competence: Apply Microprocessor and Microcontroller

Module code: ETTMM601

Credit: 8

Semester: Two

Academic Year:2025-2026B

LAB MANUAL

## Contents

Contents	2

PART ONE: MICROPROCESSOR	10

1. Introduction	10

2. GNUSim8085 Overview	10

3. General Procedure for All Experiments	10

4. Experiment List	11

Experiment 1: Study of 8085 Architecture & Instruction Set	11

Experiment 2: Addition of Two 8-bit Numbers	11

Experiment 3: Addition of Two 16-bit Numbers	12

Experiment 4: Subtraction of Two 8-bit Numbers	13

Experiment 5: Multiplication of Two 8-bit Numbers	13

Experiment 6: Division of Two 8-bit Numbers	14

Experiment 7: Logical Operations	15

Experiment 8: Find Largest of Two Numbers	16

Experiment 9: Block Data Transfer	16

Experiment 10: Sorting of Numbers	17

Experiment 11: Delay Program Using Loop	18

Experiment 12: I/O Port Programming (LED Simulation Concept)	19

6. Conclusion	19

7. References	21

PART TWO: MICROCONTROLLER	22

Experiment one: Interfacing LEDs with Arduino and controlling ON/OFF                       states	22

1.1 Objective	22

1.2 Required Components	22

1.3 Theory	22

1.4 Circuit Diagram	23

1.5 Procedure	23

1.6 Arduino Code	24

1.7 Result	25

1.8 Applications	25

1.9 Review Questions	25

1.10 Practical exercises	26

Experiment two: Interfacing multiple LEDs and control them in sequence	26

2.1 Objective	26

2.2 Required Materials	26

2.3 Theory	26

2.4 Circuit Diagram	27

2.5 Procedure	27

2.6 Arduino code	27

2.7 Result	30

2.8 Review Questions	30

2.9 Practical exercises	30

Experiment Three: Interfacing Pushbutton Control to Manually Toggle an LED	31

3.1 Objective	31

3.2 Required Materials	31

3.3 Circuit Diagram	32

3.4 Theory	33

3.5 Source Code	33

3.6 Procedure	34

3.7 Precautions	34

3.8 Review Questions	34

3.9 Practical exercises	34

Experiment Four: Use PWM to Control LED Brightness	35

4.1 Objective	35

4.2 Apparatus/Components Required	35

4.3 Theory	35

4.4 Circuit Diagram	35

4.5 Arduino Code	36

4.6 Procedure	36

4.7 Observation	37

4.8 Result	37

4.9 Conclusion	37

5.10 Safety Precautions	37

Experiment Five: Arduino Serial Communication and Control LEDs	38

5.1 Objective	38

5.2 Components Required	38

5.3 Theory	38

5.4 Circuit Diagram Description	41

5.5 Pin Connections	41

5.5.1 Send Message from Arduino to Serial Monitor	41

5.5.2 Receive Data from Serial Monitor and Control LED	42

5.6 Procedure	45

5.7 Observations	45

5.8 Review Questions	46

5.9 Conclusion	46

5.10 Practical exercises	46

Experiment Six: Interfacing Analog and Digital Sensors to Control LEDs Using Arduino	47

✅ SENSOR 1: LDR (Light Dependent Resistor)	47

6.1 Interfacing LDR Sensor to Control LED	47

6.1.1 Objective	47

6.1.2 Components Required	47

6.1.3 Circuit Diagram Description	47

6.1.4 Arduino Code	48

6.1.5 Procedure	49

6.1.6 Observations	49

6.1.7 Review  Questions	49

6.1.8 Practical exercises	49

✅ SENSOR 2: LM35 (Temperature Sensor)	50

6.2 Interfacing LM35 Sensor to Control LED	50

6.2.1 Objective	50

6.2.2 Components Required	50

6.2.3 Circuit Diagram Description	50

6.2.4 Arduino Code	51

6.2.5 Procedure	52

6.2.6 Observations	52

6.2.7 Review Questions	52

✅ SENSOR 3: Potentiometer (Analog Input)	52

6.3 Interfacing Potentiometer with Arduino to Control LED	52

6.3.1 Objective	52

6.3.2 Components Required	53

6.3.3 Circuit Diagram Description	53

6. 3.4 Arduino Code	54

6.3.5 Procedure	55

6.3.6 Observations	55

6.3.7 Review Questions	55

6.3.8 Practical exercises	55

✅ SENSOR 4: DHT11 (Temperature & Humidity Sensor)	55

6.4 Interfacing DHT11 Sensor with Arduino to Control LED	55

6.4.1 Objective	55

6.4.2 Components Required	56

6.4.3 Circuit Diagram Description	56

6. 4.4 Arduino Code	57

6.4.5 Procedure	58

6.4.6 Observations	58

6.4.7 Review Questions	58

6.4.8 Practical exercises	58

✅ SENSOR 5: DHT22 (Digital Temperature & Humidity Sensor – High Accuracy)	59

6.5 Interfacing DHT22 Sensor with Arduino to Control LED	59

6.5.1 Arduino Code	59

6.5.2 Observations	60

6.5.3 Review Questions	60

✅ SENSOR 6: Ultrasonic Sensor (HC-SR04)	60

6.6 Interfacing Ultrasonic Sensor to Control LED	60

6.6.1 Objective	60

6.6.2 Components Required	61

6.6.3 Circuit Diagram Description	61

6.6.4 Arduino Code	61

6.6.5 Procedure	63

6.6.6 Observations	63

6.6.7 Review Questions	63

6.6.8 Practical exercise	64

✅ SENSOR 7: Infrared (IR) Obstacle Sensor	64

6.7 Interfacing IR Obstacle Sensor to Control LED	64

6.7.1 Objective	64

6.7.2 Components Required	64

6.7.3 Circuit Diagram Description	65

6.7.4 Arduino Code	65

6.7.5 Procedure	66

6.7.6 Observations	66

6.7.7 Review Questions	66

6.7.8 Practical exercises	66

✅ SENSOR 8: PIR Motion Sensor	67

6.8. Interfacing PIR Motion Sensor to Control LED	67

6.8.1 Objective	67

6.8.2 Components Required	67

6.8.3 Circuit Diagram Description	67

6.8.4 Arduino Code	68

6.8.5 Procedure	69

6.8.6 Observations	69

6.8.7 Review Questions	69

6.7.8 Practical exercises	69

Experiment Seven: Interfacing Arduino Keypad to Control LEDs	69

7.1 Objective	69

7.2 Components Required	69

7.3 Circuit Diagram Description	70

7.4 Arduino Code	71

7. 5 Procedure	72

7.6 Observations	72

7.7 Review Questions	73

7.8 Conclusion	73

7.9 Practical exercise	73

Experiment Eight: Interfacing 16x2 LCD with Arduino to display messages and control LEDs	74

8.1 Objective	74

8.2 Components Required	74

8.3 Circuit Diagram Description	74

8.4 Arduino Code	75

8.5 Procedure	76

8. 6 Observations	76

8.7 Review Questions	76

Experiment Nine : Interfacing Arduino with I2C LCD Display	76

9.1 Objective	76

9.2 Components Required	76

9.3 Circuit Diagram Description	77

9.4 Arduino Code	78

9.5 Procedure	79

9.6 Observations	79

9.7 Review  Questions	79

9.8 Conclusion	79

Experiment Ten: Control LED by servo position using Arduino	80

10.1 Objective	80

10.2 Components Required	80

10.3 Pin Connections	80

10.4 Arduino Code	80

10.5 Practical exercise	81

Experiment Eleven: Interfacing Bluetooth Module (HC-05) with Arduino to Control LEDs	81

11.1 Objective	81

11.2 Components Required	81

11.3 Circuit Diagram Description	82

11.4 Arduino Code	82

11.5 Procedure	83

11.6 Observations	84

11.7 Review Questions	84

11.8 Conclusion	84

11.9 Practical exercise	84

Experiment Twelve: Interfacing Stepper motor with Arduino	85

12.1 Objective	85

12.2 Components Required	85

12.3 Circuit Diagram Description	85

12.4 Arduino Code	86

12.5 Procedure	89

12.6 Observations	89

12.7 Review Questions	89

12.8 Conclusion	90

Experiment Thirteen: Interfacing Seven Segment Display with Arduino to Indicate LED Control	90

13.1 Objective	90

13.2 Components Required	90

13.3 Circuit Diagram Description	90

13.4 Arduino Code	91

13.5 Procedure	97

13.6 Observations	97

13.7 Review Questions	97

13.8 Practical exercises	98

Experiment Fourteen: Interfacing LED Matrix/Dot matrix with arduino	98

14.1 Objective	98

14.2 Components Required	98

14.3 Circuit Diagram Description	98

14.4 Arduino Code	99

14.5 Observations	100

14.6 Review Questions	101

14.7 Conclusion	101

14.8 Practical exercises	101

Experiment Fifteen: Interfacing RFID with Arduino to Control LEDs	102

15.1 Objective	102

15.2 Components Required	102

15.3 Circuit Diagram Description	102

15.4 Arduino Code	103

15.5 Observations	105

15.6 Review Questions	105

15.7 Conclusion	105

15.8 Practical exercises	105

Experiment Sixteen: Interfacing GSM Module to Control LEDs via SMS	106

16.1 Objective	106

16.2 Components Required	106

16.3 Circuit Diagram Description	107

16.3 Arduino Code	107

16.4 Procedure	108

16.5 Observations	108

16.6 Review Questions	109

16.7 Conclusion	109

# PART ONE: MICROPROCESSOR

Intel 8085 Microprocessor Experiments using GNUSim8085 Simulator

# 1. Introduction

The Intel 8085 is an 8-bit microprocessor widely used for teaching microprocessor fundamentals. This lab manual is designed to help students perform standard 8085 microprocessor experiments using the GNUSim8085 simulator, eliminating the need for physical hardware while preserving core learning outcomes.

Objectives

- Understand the architecture and instruction set of the 8085 microprocessors.

- Write, assemble, and execute 8085 assembly language programs

- Analyze register, memory, and flag operations

- Simulate I/O operations using ports (LEDs, switches conceptually)

Software Required

- GNUSim8085 Simulator (Windows / Linux)

# 2. GNUSim8085 Overview

GNUSim8085 is a free, open-source graphical simulator for the Intel 8085 microprocessor.

Key Features

- Integrated editor and assembler

- Step-by-step execution

- Register, flag, and memory view

- Breakpoints and debugging

- I/O port simulation

# 3. General Procedure for All Experiments

- Open GNUSim8085

- Create a new source file

- Write the assembly language program

- Assemble the program (Assemble → Assemble)

- Load the program into memory

- Execute using:

  - Run

  - Step Execution

- Observe:

  - Registers

  - Flags

  - Memory

  - I/O ports

# 4. Experiment List

# Experiment 1: Study of 8085 Architecture & Instruction Set

Aim: To study the architecture and instruction set of the Intel 8085 microprocessor.

Theory: The 8085 consists of ALU, registers, control unit, and buses. Instructions are categorized as data transfer, arithmetic, logical, branching, and machine control instructions.

Procedure:

- Identify registers and flags in GNUSim8085

- Observe PC, SP, and flag changes during execution

# Experiment 2: Addition of Two 8-bit Numbers

Aim: To add two 8-bit numbers stored in registers.

Algorithm:

- Start the program.

- Load the first 8-bit number into the accumulator (A).

- Load the second 8-bit number into another register (B).

- Add the contents of register B to the accumulator using ADD instruction.

- Store the result in the accumulator.

- Check the Carry flag for overflow.

- Stop the program.

Program:

MVI A, 25H

MVI B, 34H

ADD B

OUT 08H

HLT

Observation:

- Accumulator contains sum

- Carry flag indicates overflow

# Experiment 3: Addition of Two 16-bit Numbers

Aim: To add two 16-bit numbers using register pairs.

Algorithm:

- Start the program.

- Load the first 16-bit number into HL register pair.

- Load the second 16-bit number into DE register pair.

- Add DE register pair to HL using DAD instruction.

- Observe the result in HL register pair.

- Check Carry flag for overflow.

- Stop the program.

Program:

LXI H, 1234H

LXI D, 4321H

DAD D

HLT

# Experiment 4: Subtraction of Two 8-bit Numbers

Aim: To subtract one 8-bit number from another.

Algorithm:

- Start the program.

- Load the minuend into the accumulator.

- Load the subtrahend into another register.

- Subtract the register value from accumulator using SUB instruction.

- Observe the result in the accumulator.

- Check Carry flag for borrow.

- Stop the program.

Program:

MVI A, 50H

MVI B, 25H

SUB B

HLT

Observation: Borrow reflected in carry flag.

# Experiment 5: Multiplication of Two 8-bit Numbers

Aim: To multiply two 8-bit numbers using repeated addition.

Algorithm:

- Start the program.

- Load the multiplicand into register B.

- Load the multiplier into register C.

- Clear accumulator and register pair HL.

- Add multiplicand to accumulator.

- Decrement counter.

- Repeat until counter becomes zero.

- Store result in HL register pair.

- Stop the program.

Program:

MVI B, 05H ; Multiplicand

MVI C, 04H ; Multiplier

MVI A, 00H

LXI H, 0000H

LOOP: ADD B

JNC NEXT

INX H

NEXT: DCR C

JNZ LOOP

MOV L, A

HLT

Algorithm:

- Load numbers

- Repeatedly add until counter reaches zero

# Experiment 6: Division of Two 8-bit Numbers

Aim: To divide two 8-bit numbers using repeated subtraction.

Algorithm:

- Start the program.

- Load dividend into accumulator.

- Load divisor into register B.

- Clear register C for quotient.

- Subtract divisor repeatedly.

- Increment quotient count.

- Stop when remainder is less than divisor.

Program:

MVI A, 14H ; Dividend

MVI B, 04H ; Divisor

MVI C, 00H ; Quotient

LOOP: CMP B

JC END

SUB B

INR C

JMP LOOP

END: HLT

# Experiment 7: Logical Operations

Aim: To perform AND, OR, XOR logical operations.

Algorithm:

- Start the program.

- Load first data into accumulator.

- Load second data into another register.

- Perform logical operation (ANA / ORA / XRA).

- Observe result and flag status.

- Stop the program.

Program:

MVI A, 55H

MVI B, 0FH

ANA B

HLT

# Experiment 8: Find Largest of Two Numbers

Aim: To find the largest of two 8-bit numbers.

Algorithm:

- Start the program.

- Load first number into accumulator.

- Load second number into register B.

- Compare numbers using CMP.

- Store larger number.

- Stop the program.

Program:

MVI A, 25H

MVI B, 30H

CMP B

JC B_LARGE

MOV C, A

HLT

B_LARGE: MOV C, B

HLT

# Experiment 9: Block Data Transfer

Aim: To transfer a block of data from one memory location to another.

Algorithm:

- Start the program.

- Load source address into HL.

- Load destination address into DE.

- Load count into register C.

- Transfer data byte-by-byte.

- Stop the program.

Program:

LXI H, 2050H

LXI D, 3050H

MVI C, 05H

LOOP: MOV A, M

STAX D

INX H

INX D

DCR C

JNZ LOOP

HLT

# Experiment 10: Sorting of Numbers

Aim: To sort numbers in ascending order.

Algorithm:

- Start the program.

- Load count.

- Compare adjacent numbers.

- Swap if required.

- Repeat passes.

- Stop the program.

Program (Bubble Sort – 3 Numbers):

LXI H, 2050H

MVI C, 03H

OUTER: DCR C

JZ END

MOV D, C

LXI H, 2050H

INNER: MOV A, M

INX H

CMP M

JC SKIP

MOV B, M

MOV M, A

DCX H

MOV M, B

INX H

SKIP: DCR D

JNZ INNER

JMP OUTER

END: HLT

# Experiment 11: Delay Program Using Loop

Aim: To generate a time delay using loop instructions.

Algorithm:

- Start the program.

- Load count into register pair.

- Decrement continuously.

- Stop when zero.

Program:

LXI B, FFFFH

LOOP: DCX B

MOV A, B

ORA C

JNZ LOOP

HLT

# Experiment 12: I/O Port Programming (LED Simulation Concept)

Aim: To simulate LED control using output ports.

Algorithm:

- Start the program.

- Load LED pattern into accumulator.

- Output to port.

- Stop program.

Program:

MVI A, 0AAH; LED pattern

OUT 01H

HLT

Observation: Port 01H shows output pattern representing LED ON/OFF.

Example 1: Turn ON all LEDs

MVI A, FFH

OUT 01H

HLT

//FFH = 11111111 → all LEDs ON.

Example 2: Turn OFF all LEDs

MVI A, 00H

OUT 01H

HLT

//00H = 00000000 → all LEDs OFF.

Example 3: Turn ON alternate LEDs

MVI A, AAH

OUT 01H

HLT

//AAH = 10101010

These lights alternate LEDs.

Example 4: Running LED Pattern

MVI A, 01H

START:

OUT 01H

RLC

JMP START

This rotates the bit and creates moving LEDs.

Adding a delay subroutine between OUT and RLC make the LEDs to move too fast because the 8085 executes instructions very quickly.

MVI A,01H

LOOP:

OUT 01H

CALL DELAY

RLC

JMP LOOP

DELAY:

MVI B, FFH

D1:

DCR B

JNZ D1

RET

How it works

- OUT 01H → sends data to LEDs

- CALL DELAY → pauses for some time

- RLC → rotates LED bit

- JMP LOOP → repeats forever

Result

The LEDs will now move slowly one by one.

To Make It Slower

Increase the delay using nested loops:

MVI A,01H

LOOP:

OUT 01H

CALL DELAY

RLC

JMP LOOP

DELAY:

MVI B, FFH

L1:

MVI C, FFH

L2:

DCR C

JNZ L2

DCR B

JNZ L1

RET

This creates a much better visible LED running effect in Sim8085.

If you want LEDs to turn ON and OFF repeatedly (blinking), use these 8085 programs:

START:

MVI A,FFH     ; All LEDs ON

OUT 01H

CALL DELAY

MVI A,00H     ; All LEDs OFF

OUT 01H

CALL DELAY

JMP START

DELAY:

MVI B,FFH

L1:

MVI C,FFH

L2:

DCR C

JNZ L2

DCR B

JNZ L1

RET

What happens

- FFH → turns all LEDs ON

- 00H → turns all LEDs OFF

- Delay makes blinking visible

- JMP START repeats forever

Result

LEDs will:

ON → OFF → ON → OFF → ... continuously in the LED Array.

The program is running forever because of: JMP START

To see the LED ON/OFF blinking slowly, you should:

- Output one value → LEDs ON

- Add a BIG delay

- Output another value → LEDs OFF

- Add another BIG delay

- Repeat

START:

; LEDs ON

MVI A, 0FFH

OUT 01H

CALL DELAY

; LEDs OFF

MVI A, 00H

OUT 01H

CALL DELAY

JMP START

; -------------------------

; LONG DELAY SUBROUTINE

; -------------------------

DELAY:

MVI B, 0FFH

DELAY1:

MVI C, 0FFH

DELAY2:

DCR C

JNZ DELAY2

DCR B

JNZ DELAY1

RET

What this does:

- 0FFH → all LEDs ON

- 00H → all LEDs OFF

- Large nested loops create a slow visible blink

- CALL DELAY makes the interval high

To blink LEDS n-times

START:

; -------------------------

; BLINK LED1 FIVE TIMES

; -------------------------

MVI D, 05H

LED1:

MVI A, 01H     ; LED 1 ON

OUT 01H

CALL DELAY

MVI A, 00H     ; LED OFF

OUT 01H

CALL DELAY

DCR D

JNZ LED1

; -------------------------

; BLINK LED2 FIVE TIMES

; -------------------------

MVI D, 05H

LED2:

MVI A, 02H

OUT 01H

CALL DELAY

MVI A, 00H

OUT 01H

CALL DELAY

DCR D

JNZ LED2

; -------------------------

; BLINK LED3 FIVE TIMES

; -------------------------

MVI D, 05H

LED3:

MVI A, 04H

OUT 01H

CALL DELAY

MVI A, 00H

OUT 01H

CALL DELAY

DCR D

JNZ LED3

; -------------------------

; BLINK LED4 FIVE TIMES

; -------------------------

MVI D, 05H

LED4:

MVI A, 08H

OUT 01H

CALL DELAY

MVI A, 00H

OUT 01H

CALL DELAY

DCR D

JNZ LED4

JMP START

; =================================

; LONG DELAY SUBROUTINE

; =================================

DELAY:

MVI B, 0FFH

DELAY1:

MVI C, 0FFH

DELAY2:

DCR C

JNZ DELAY2

DCR B

JNZ DELAY1

RET

EXPLANATION OF THE CODE:

START:

- This is a label.

- Program execution starts from here.

- START is used later with JMP START.

MVI D, 05H

- MVI = Move Immediate data.

- Load register D with 05H.

- 05H means decimal 5.

- Register D acts as the counter for blinking 5 times.

LED1 SECTION

LED1:

- Label for LED1 blinking loop.

MVI A, 01H

- Load accumulator A with 01H.

- Binary value: 00000001

- This turns ON only the first LED.

OUT 01H

- Send contents of accumulator A to output port 01H.

- LED array connected to port 01H.

- LED1 glows.

CALL DELAY

- Calls the delay subroutine.

- Program jumps to DELAY.

- Creates visible ON time.

MVI A, 00H

- Load 00H into accumulator.

- Binary:00000000

- All LEDs OFF.

OUT 01H

- Output 00H to port.

- LED1 turns OFF.

CALL DELAY

- Wait again.

- Creates visible OFF time.

DCR D

- DCR = Decrement register.

- Reduce counter D by 1.

Example:

05 → 04 → 03 → 02 → 01 → 00

JNZ LED1

- JNZ = Jump if Not Zero.

- If D ≠ 0, go back to LED1.

- Repeats blinking.

When D = 0, execution moves to next section.

LED2 SECTION

MVI D, 05H

- Reload counter with 5.

LED2:

- Label for LED2 loop.

MVI A, 02H

- Binary:00000010

- Turns ON second LED.

OUT 01H

CALL DELAY

- LED2 ON and wait.

MVI A, 00H

OUT 01H

CALL DELAY

- LED2 OFF and wait.

DCR D

JNZ LED2

- Repeat LED2 blinking 5 times.

LED3 SECTION

MVI D, 05H

- Reset counter.

LED3:

MVI A, 04H

Binary:00000100

- Turns ON third LED.

OUT 01H

CALL DELAY

MVI A, 00H

OUT 01H

CALL DELAY

DCR D

JNZ LED3

- Blink LED3 five times.

LED4 SECTION

MVI D, 05H

- Counter reset.

LED4:

MVI A, 08H

Binary: 00001000

- Turns ON fourth LED.

OUT 01H

CALL DELAY

MVI A, 00H

OUT 01H

CALL DELAY

DCR D

JNZ LED4

- Blink LED4 five times.

REPEAT PROGRAM

JMP START

- Unconditional jump.

- Program repeats forever from beginning.

DELAY SUBROUTINE

DELAY:

- Label for delay routine.

MVI B, 0FFH

- Load register B with maximum value 255.

- Outer delay loop counter.

DELAY1:

- Outer loop label.

MVI C, 0FFH

- Load register C with 255.

- Inner delay loop counter.

DELAY2:

- Inner loop label.

DCR C

- Decrease C by 1.

JNZ DELAY2

- Repeat inner loop until C = 0.

This creates small delay.

DCR B

- After inner loop completes,

- Decrease outer counter B.

JNZ DELAY1

- Repeat whole inner loop again.

- Creates long delay.

RET

- Return from subroutine.

- Control goes back to instruction after CALL DELAY.

Overall Working

Sequence:

LED1 blinks 5 times

↓

LED2 blinks 5 times

↓

LED3 blinks 5 times

↓

LED4 blinks 5 times

↓

Program repeats forever

START:

; =================================

; LED1 BLINK 3 TIMES

; =================================

MVI D, 03H

LED1:

MVI A, 01H

OUT 01H

CALL DELAY

MVI A, 00H

OUT 01H

CALL DELAY

DCR D

JNZ LED1

; =================================

; LED2 BLINK 5 TIMES

; =================================

MVI D, 05H

LED2:

MVI A, 02H

OUT 01H

CALL DELAY

MVI A, 00H

OUT 01H

CALL DELAY

DCR D

JNZ LED2

; =================================

; LED3 BLINK 6 TIMES

; =================================

MVI D, 06H

LED3:

MVI A, 04H

OUT 01H

CALL DELAY

MVI A, 00H

OUT 01H

CALL DELAY

DCR D

JNZ LED3

; =================================

; LED4 BLINK 9 TIMES

; =================================

MVI D, 09H

LED4:

MVI A, 08H

OUT 01H

CALL DELAY

MVI A, 00H

OUT 01H

CALL DELAY

DCR D

JNZ LED4

; =================================

; REPEAT FOREVER

; =================================

JMP START

; =================================

; DELAY SUBROUTINE

; =================================

DELAY:

MVI B, 0FFH

DELAY1:

MVI C, 0FFH

DELAY2:

DCR C

JNZ DELAY2

DCR B

JNZ DELAY1

RET

START:

; =================================

; ALL LEDs BLINK 4 TIMES

; =================================

MVI D, 04H

ALL_LED:

; ALL LEDs ON

MVI A, 0FFH

OUT 01H

CALL DELAY

; ALL LEDs OFF

MVI A, 00H

OUT 01H

CALL DELAY

DCR D

JNZ ALL_LED

; =================================

; LED1 BLINK 3 TIMES

; =================================

MVI D, 03H

LED1:

MVI A, 01H

OUT 01H

CALL DELAY

MVI A, 00H

OUT 01H

CALL DELAY

DCR D

JNZ LED1

; =================================

; LED2 BLINK 5 TIMES

; =================================

MVI D, 05H

LED2:

MVI A, 02H

OUT 01H

CALL DELAY

MVI A, 00H

OUT 01H

CALL DELAY

DCR D

JNZ LED2

; =================================

; LED3 BLINK 6 TIMES

; =================================

MVI D, 06H

LED3:

MVI A, 04H

OUT 01H

CALL DELAY

MVI A, 00H

OUT 01H

CALL DELAY

DCR D

JNZ LED3

; =================================

; LED4 BLINK 9 TIMES

; =================================

MVI D, 09H

LED4:

MVI A, 08H

OUT 01H

CALL DELAY

MVI A, 00H

OUT 01H

CALL DELAY

DCR D

JNZ LED4

; =================================

; REPEAT FOREVER

; =================================

JMP START

; =================================

; DELAY SUBROUTINE

; =================================

DELAY:

MVI B, 0FFH

DELAY1:

MVI C, 0FFH

DELAY2:

DCR C

JNZ DELAY2

DCR B

JNZ DELAY1

RET

Compare Two Numbers Using Registers and Turn ON LED if the Number is Larger

; =================================

; COMPARE TWO NUMBERS USING REGISTERS

; NUMBERS: 23H and 35H

; TURN ON LED IF 35H IS LARGER

; =================================

START:

MVI B, 23H       ; Load first number into B

MVI A, 35H       ; Load second number into A

CMP B            ; Compare A with B

JC FIRST_BIG     ; If A < B jump

; =================================

; SECOND NUMBER IS LARGER

; =================================

MVI A, 0FFH      ; ALL LEDs FOR PORT1 WILL BE ON

OUT 01H

HLT

; =================================

; FIRST NUMBER IS LARGER

; =================================

FIRST_BIG:

MVI A, 01H       ; ONE LED FOR PORT8 WILL BE ON

OUT 08H

HLT

Working

- 23H stored in register B

- 35H stored in accumulator A

- CMP B compares: A - B

Since:

35H > 23H

ALL LEDs turn ON using:

MVI A,0FFH

OUT 01H

# 6. Conclusion

This lab manual enables effective understanding of 8085 microprocessor concepts using GNUSim8085. It replaces hardware-based experiments with a safe, flexible, and repeatable simulation-based approach.

N.B: Addition with carry if any

Add Two 8-bit Numbers (8085 Assembly Language)

Problem Statement

Add two 8-bit numbers stored in memory locations 2050H and 2051H.
Store the sum at 2052H and the carry (if any) at 2053H.

Algorithm

- Load the first 8-bit number into the accumulator.

- Add the second 8-bit number to the accumulator.

- Store the sum in memory.

- Check carry flag.

- If carry exists, store 01H; otherwise store 00H.

- Stop execution.

8085 Assembly Language Program

LDA 2050H      ; Load first number into A

MOV B, A       ; Save it in register B

LDA 2051H      ; Load second number into A

ADD B          ; A = A + B

STA 2052H      ; Store sum at 2052H

JNC NO_CARRY   ; Jump if no carry

MVI A, 01H     ; Carry = 1

STA 2053H      ; Store carry

JMP END

NO_CARRY:

MVI A, 00H     ; Carry = 0

STA 2053H

END:

HLT            ; Stop execution

Explanation

- LDA loads data from memory into the accumulator.

- ADD performs 8-bit addition.

- Carry is checked using the CY flag.

- JNC jumps if carry flag = 0.

- Carry result is stored separately.

Example Input and Output

| Memory Location | Value (Hex) |
| --- | --- |
| 2050H | 8AH |
| 2051H | 79H |

Result:

- Sum at 2052H → 03H

- Carry at 2053H → 01H

Simpler Version (Without Carry Storage)

LDA 2050H

ADD M          ; If HL points to 2051H

STA 2052H

HLT

# 7. References

- Intel 8085 Microprocessor User Manual

- GNUSim8085 Documentation

- Ramesh Gaonkar – Microprocessor Architecture, Programming and Applications

# PART TWO: MICROCONTROLLER

# Experiment one: Interfacing LEDs with Arduino and controlling ON/OFF                       states

## 1.1 Objective

To interface one or more LEDs with an Arduino microcontroller and control their ON/OFF states through programming using Arduino IDE.

## 1.2 Required Components

| Component | Quantity |
| --- | --- |
| Arduino Uno (or compatible board) | 1 |
| LED (any color) | 1–3 |
| 220-ohm resistor | 1–3 |
| Breadboard | 1 |
| Jumper wires | As required |
| USB Cable (for Arduino) | 1 |
| Computer with Arduino IDE installed | 1 |

## 1.3 Theory

An LED (Light Emitting Diode) is a semiconductor device that emits light when current passes through it. Arduino can control the state (ON/OFF) of an LED through its digital output pins using programming logic (HIGH for ON, LOW for OFF).

## 1.4 Circuit Diagram

## 1.5 Procedure

✅ Hardware Setup:

- Place the LED on the breadboard.

- Connect the anode (long leg) of the LED to Digital Pin 13 of Arduino through a 220Ω resistor.

- Connect the cathode (short leg) to the GND on Arduino.

- Connect Arduino to the computer using the USB cable.

✅ Software Setup:

- Open Arduino IDE on your computer.

- Select the correct board type: Tools → Board → Type of arduino board(Arduino Uno)

- Select the correct COM port: Tools → Port → COMx

- Write arduino code

- Compile and upload the code to microcontroller IC using the Compile/ verify and Upload buttons.

For more details on above figure, click on the link below

https://www.makerspaces.com/simple-arduino-projects-beginners/

Exercise: From figure below, name the numbers 1 to 12 with their role

## 1.6 Arduino Code

int ledPin = 13; // Set digital pin for LED

void setup() {

pinMode(ledPin, OUTPUT); // Set pin as OUTPUT

}

void loop() {

digitalWrite(ledPin, HIGH); // Turn ON LED

delay(1000);                // Wait for 1 second

digitalWrite(ledPin, LOW);  // Turn OFF LED

delay(1000);                // Wait for 1 second

}

## 1.7 Result

- LED successfully turns ON and OFF repeatedly.

- Code execution verified the ability to interface and control LEDs with Arduino.

## 1.8 Applications

- Basic building block for larger embedded systems projects.

- Visual indicators in automation (status, alerts, feedback).

- Educational tool for beginners learning microcontroller I/O.

## 1.9 Review Questions

- What is the function of pinMode() in Arduino?

- Why do we use a resistor with an LED?

- What is the difference between digitalWrite() and analogWrite()?

- Can you connect an LED without a resistor? What may happen?

- How would you control multiple LEDs independently?

## 1.10 Practical exercises

1. As an electronic technician, you are requested to provide an electronic LED flashing product with one LED in the way that the loop consists of red LED  stays turning ON in eight seconds and OFF in 2seconds.

a.	Draw clearly the design circuit for above situation in proteus software

b.	Develop correctly  the code for the design circuit

c.	Simulate properly the designed circuit.

# Experiment two: Interfacing multiple LEDs and control them in sequence

## 2.1 Objective

To design and implement a system that interfaces multiple LEDs with a microcontroller and programs them to create a chaser light (running light) effect.

## 2.2 Required Materials

| No. | Component | Quantity |
| --- | --- | --- |
| 1. | Arduino Uno / Atmega328 board | 1 |
| 2. | LEDs (Red/Green) | 8 |
| 3. | Resistors (220Ω or 330Ω) | 8 |
| 4. | Breadboard | 1 |
| 5. | Jumper Wires | As needed |
| 6. | USB Cable / Power Supply | 1 |
| 7. | Computer with Arduino IDE | 1 |

## 2.3 Theory

The LED chaser effect is a sequence where multiple LEDs light up one after the other to create a visual "moving" effect. It is commonly used in decorative lighting, indicators, and embedded system learning. This is achieved by programming digital output pins to toggle the LEDs on and off in a timed sequence.

## 2.4 Circuit Diagram

## 2.5 Procedure

- Connect the anode of each LED to Arduino digital pins D9 to D11 through resistors.

- Connect the cathode of each LED to GND.

- Power up the Arduino using a USB cable.

- Open Arduino IDE, create a new sketch.

- Write and upload the program (see below).

- Observe the LEDs blinking one after the other in a chaser pattern.

## 2.6 Arduino code

Arduino  Code 1: Turn ON three LEDs one by one and OFF  one by one.

// Define LED pins

int led1 = 9;

int led2 = 10;

int led3 = 11;

void setup() {

// Set all LED pins as OUTPUT

pinMode(led1, OUTPUT);

pinMode(led2, OUTPUT);

pinMode(led3, OUTPUT);}

void loop() {

// Turn ON all LEDs one by one

digitalWrite(led1, HIGH);

delay(500); // wait 0.5 seconds

digitalWrite(led2, HIGH);

delay(500);

digitalWrite(led3, HIGH);

delay(1000); // wait 1 second

// Turn OFF all LEDs one by one

digitalWrite(led1, LOW);

delay(500);

digitalWrite(led2, LOW);

delay(500);

digitalWrite(led3, LOW);

delay(1000);}

Arduino code 2: Four LEDs ON and OFF

int leds [ ] = {6, 7, 8, 9};

void setup() {

for (int i = 0; i < 4; i++) {

pinMode(leds[i], OUTPUT);  }}

void loop() {

for (int i = 0; i < 4; i++) {

digitalWrite(leds[i], HIGH);

delay(300);  }

for (int i = 0; i < 4; i++) {

digitalWrite(leds[i], LOW);

delay(300);  }}

Arduino  Code 3 : eight LEDs with array

const int ledCount = 8;

int leds[ledCount] = {2, 3, 4, 5, 6, 7, 8, 9};

void setup() {

for (int i = 0; i < ledCount; i++) {

pinMode(leds[i], OUTPUT); }}

void loop() {

for (int i = 0; i < ledCount; i++) {

digitalWrite(leds[i], HIGH);

delay(100);

digitalWrite(leds[i], LOW);  }

for (int i = ledCount - 2; i > 0; i--) { // reverse

digitalWrite(leds[i], HIGH);

delay(100);

digitalWrite(leds[i], LOW);  }}

## 2.7 Result

The LEDs light up sequentially from left to right and then in reverse (right to left), successfully demonstrating the chaser effect.

## 2.8 Review Questions

- What is the purpose of using resistors with LEDs?

- How does delay() affect the LED chaser speed?

- What modifications are required to make the sequence faster?

- What is the difference between active HIGH and LOW LEDs?

🔄 Variations to Try

- Make the LEDs blink in reverse order

- Blink all LEDs simultaneously

📄 Conclusion

This experiment introduces the core principles of microcontroller-based digital output. Students gain hands-on experience in hardware interfacing and efficient programming techniques like arrays and loops.

## 2.9 Practical exercises

1. As an electronic technician, you are requested to provide an electronic LEDs flashing product having four LEDs in the way that the loop consists of red LED turning ON/OFF six times for 200ms, blue LED turning ON/OFF four times for 400ms, yellow LED turning ON/OFF three times for 600ms and green LED turning ON/OFF two times for 800ms.

a.	Draw clearly the design circuit for above situation in proteus software

b.	Develop correctly  the code for the design circuit

c.	Simulate properly the designed circuit.

2. A single way traffic light is designed in the way red lamps lights 7seconds to stop vehicles to pass, green lamp to allow vehicles to pass during 10 seconds and yellow lamp is using for driving slowly and the time allocation for this lamp is 3 seconds. Note that Off-time is 10th of a second. As electronic technician, you are requested to write code and simulate it in Proteus. Do the same for four-way traffic light.

3. A single way traffic light is designed in the way red lamps lights 7seconds to stop vehicles to pass and blinks 3 times to alert the drivers that the time to stop is going to end, green lamp to allow vehicles to pass during 10 seconds and blinks 3 times to alert also the drivers that the time to pass is going to finish and yellow lamp is using for driving slow and the time allocation for this lamp is 3 seconds. Note that Off-time is 10th of a second and period of blinking is 300ms. As electronic technician, you are requested to write code and simulate it in Proteus

4.  Write the code and make the simulation using Proteus software to turn ON and OFF 8 LEDs depend on the following sequences:

a.	All LEDS turn ON (1second) and OFF (1second) five times.

b.	LEDs ON (500ms) and OFF (500ms) from left to right one by one.

c.	LEDs ON (300ms) and OFF (300ms) from right to left one by one.

d.	All LEDS turn ON (1second) and OFF (1second) five times from left one by one

5. As an electronic technician, you are requested to design and simulate the LEDs in the form of heart (use 12 LEDs of different available colors) in the way that All LEDS turn on -off five times, all LEDs turn on-off one by one from left to right, then all LEDs turns on-off one by one from right to left by jumping one LED and lastly each LED blinks three times from right to left.

# Experiment Three: Interfacing Pushbutton Control to Manually Toggle an LED

## 3.1 Objective

To design and implement a simple embedded system circuit where a pushbutton switch is used to manually toggle an LED ON and OFF using a microcontroller (e.g., Arduino or ATmega328).

## 3.2 Required Materials

| Component | Quantity |
| --- | --- |
| Microcontroller (e.g., Arduino Uno or ATmega328) | 1 |
| LED | 1 |
| Pushbutton | 1 |
| Resistors (220Ω for LED, 10kΩ for pull-down) | 2 |
| Breadboard | 1 |
| Jumper Wires | As needed |
| USB Cable (for Arduino) | 1 |
| Power Supply (if needed) | 1 |

## 3.3 Circuit Diagram

Connections:

- LED: Connect the anode (+) of the LED to digital pin 13 (or any GPIO pin) via a 220Ω resistor, and the cathode (-) to GND.

- Pushbutton:

  - Connect one leg of the pushbutton to digital pin 2.

  - Connect the same leg to GND via a 10kΩ pull-down resistor.

  - Connect the opposite leg to 5V.

This sets up the button to read LOW when not pressed, and HIGH when pressed.

## 3.4 Theory

A pushbutton is a digital input device. When pressed, it completes the circuit and sends a signal to the microcontroller. In this lab, we use it to toggle the state of an LED. That means:

- First press → LED ON

- Second press → LED OFF

- Third press → LED ON again, and so on…

To detect a toggle, we check for state change (edge detection), not just level.

## 3.5 Source Code

const int buttonPin = 3;

const int ledPin = 11;

bool ledState = false;

bool lastButtonState = HIGH;

void setup() {

pinMode(buttonPin, INPUT_PULLUP);  // Enable internal pull-up resistor

pinMode(ledPin, OUTPUT);}

void loop() {

bool buttonState = digitalRead(buttonPin);

// Check for button press (active LOW) and detect edge

if (buttonState == LOW && lastButtonState == HIGH) {

ledState = !ledState;  // Toggle LED state

digitalWrite(ledPin, ledState);

delay(200); // debounce delay

}

lastButtonState = buttonState;

}

## 3.6 Procedure

- Assemble the components on the breadboard as per the circuit diagram.

- Upload the Arduino code to the board using the Arduino IDE.

- Observe the LED:

  - Press the button once → LED turns ON.

  - Press again → LED turns OFF.

- Continue testing the toggle behavior.

## 3.7 Precautions

- Ensure correct resistor values to protect the LED.

- Use a pull-down resistor to prevent floating input states.

- Debounce the button properly to avoid multiple toggles from a single press.

## 3.8 Review Questions

- What is the purpose of the pull-down resistor?

- Why do we use debouncing?

- What would happen if we didn't use debouncing?

- How can this logic be extended to multiple buttons?

- What is the difference between edge detection and level detection?

## 3.9 Practical exercises

1. Write and simulate the code to turn on and off all two LEDs (blue and yellow) three times when the switch is pressed and to turn On a LED (green) when the switch is released.

2.

# Experiment Four: Use PWM to Control LED Brightness

## 4.1 Objective

To learn how to use Pulse Width Modulation (PWM) to control the brightness of an LED using a microcontroller (e.g., Arduino UNO).

## 4.2 Apparatus/Components Required

| Component | Quantity |
| --- | --- |
| Arduino Uno | 1 |
| Breadboard | 1 |
| LED (any color) | 1 |
| Resistor (220Ω) | 1 |
| Jumper wires | As required |
| USB Cable | 1 |
| Computer with Arduino IDE | 1 |

## 4.3 Theory

Pulse Width Modulation (PWM) is a technique where the amount of power delivered to a device is controlled by switching it on and off very quickly. The average voltage (and power) depends on the duty cycle of the signal.

- Duty Cycle: The percentage of time the signal is "HIGH" in one cycle.

- In Arduino, PWM is generated using the analogWrite(pin, value) function, where value ranges from 0 (off) to 255 (fully on).

## 4.4 Circuit Diagram

Connection:

- One end of the resistor is connected to the digital pin 9.

- The other end goes to the anode (long leg) of the LED.

- The cathode (short leg) of the LED goes to GND.

## 4.5 Arduino Code

int ledPin = 9; // PWM-capable pin

void setup() {

pinMode(ledPin, OUTPUT);}

void loop() {

// Increase brightness

for (int brightness = 0; brightness <= 255; brightness++) {

analogWrite(ledPin, brightness);

delay(10);  }

// Decrease brightness

for (int brightness = 255; brightness >= 0; brightness--) {

analogWrite (ledPin, brightness);

delay(10); }}

## 4.6 Procedure

- Connect the circuit as shown in the diagram.

- Plug in the Arduino board to your computer via USB.

- Open the Arduino IDE.

- Write the code above into the sketch.

- Click Upload to flash the code to your Arduino.

- Observe the LED fading in and out smoothly.

## 4.7 Observation

| Duty Cycle (%) | Brightness Level | Description |
| --- | --- | --- |
| 0 | OFF | LED is off |
| 25 | Dim | Low brightness |
| 50 | Medium | Medium brightness |
| 75 | High | Brighter |
| 100 | Max | Fully bright (constant) |

## 4.8 Result

The brightness of the LED varies smoothly depending on the PWM duty cycle. The LED appears brighter at higher duty cycles.

## 4.9 Conclusion

PWM is a simple yet powerful technique to simulate analog voltage levels using digital output. It’s widely used in LED dimming, motor control, and signal modulation.

5.10 Safety Precautions

- Ensure LED polarity is correct.

- Don’t exceed 5V or 20mA on any digital pin.

- Use current-limiting resistors to protect the LED.

# Experiment Five: Arduino Serial Communication and Control LEDs

## 5.1 Objective

To understand how to use serial communication between a computer and Arduino to control LEDs using the Serial Monitor.

## 5.2 Components Required

| Component | Quantity |
| --- | --- |
| Arduino Uno | 1 |
| LEDs (Red, Yellow, Green, Blue) | 4 |
| 220Ω Resistors | 4 |
| Breadboard | 1 |
| Jumper wires | As needed |
| USB cable | 1 |
| Laptop with Arduino IDE | 1 |

## 5.3 Theory

Serial communication is the process of sending data bit by bit over a single communication line. In Arduino, the most commonly used method is UART (Universal Asynchronous Receiver/Transmitter), which uses the TX (transmit) and RX (receive) pins.

Arduino serial communication refers to the process by which an Arduino board exchanges data with other devices, such as a computer, another microcontroller, or a peripheral, using a serial communication protocol. This involves sending and receiving data bit by bit over a single line.

Key aspects of Arduino Serial Communication:

Physical Components:

Serial Port: Arduino boards typically have at least one serial port, often referred to as Serial, which utilizes digital pins 0 (RX - Receive) and 1 (TX - Transmit). Some boards, like the Mega 2560 or Due, have additional serial ports (e.g., Serial1, Serial2).

USB Port: On USB-enabled boards (e.g., Uno, Leonardo), the USB port also facilitates serial communication with a computer, appearing as a virtual serial port.

Software Components:

Arduino IDE Serial Monitor: The Arduino Integrated Development Environment (IDE) includes a Serial Monitor tool, which allows users to send data to the Arduino and view data received from the Arduino, facilitating debugging and interaction.

Serial Library Functions: The Arduino programming language provides a built-in Serial library with functions for managing serial communication:

Serial.begin(baud rate): Initializes serial communication at a specified baud rate (data transfer speed).

Baud rate defines how fast data is transmitted (commonly 9600 bps).

Serial.print() / Serial.println(): Sends data as human-readable characters to the serial port.

Serial.print(value) and Serial.println(value): Used to send data (text or numerical values) from the Arduino to the serial monitor or another device. println adds a newline character after the data.

Serial.write(): Sends raw binary data (bytes) to the serial port.

Serial.available(): Checks if there is incoming serial data to read.

Serial.read(): Reads incoming serial data.

How it works:

Initialization:

The Serial.begin() function sets up the serial communication parameters, including the baud rate, ensuring both devices communicate at the same speed.

Data Transmission:

When Serial.print() or Serial.write() is used, the Arduino converts the data into a stream of bits and sends them sequentially through the TX pin.

Data Reception:

When data is sent to the Arduino's RX pin, the Arduino receives the bits, reassembles them, and makes them available for reading using Serial.read().

Debugging and Interaction:

The Serial Monitor in the Arduino IDE provides a visual interface to observe the data being sent and received, enabling debugging and real-time interaction with the Arduino.

Applications:

Serial communication is fundamental for various Arduino applications, including:

- Sending sensor readings to a computer for logging or analysis.

- Receiving commands from a computer or another device to control actuators or change program behavior.

- Interacting with other microcontrollers or serial-enabled modules (e.g., GPS modules, Bluetooth modules).

🔌 Arduino Pins Used:

- TX (Digital Pin 1): Transmit data

- RX (Digital Pin 0): Receive data
However, for computer communication via USB, we use the built-in Serial interface via the USB port.

## 5.4 Circuit Diagram Description

- Connect each LED (with its 220Ω resistor) to a separate digital pin on the Arduino (e.g., D10 to D13).

- The Serial Monitor will be used to type commands to turn ON/OFF each LED.

Note: No external serial module needed. Serial communication is done via USB.

## 5.5 Pin Connections

LED Pins → Arduino

| LED Color | Arduino Pin | Resistor |
| --- | --- | --- |
| Red | D10 | 220Ω |
| Yellow | D11 | 220Ω |
| Green | D12 | 220Ω |
| Blue | D13 | 220Ω |

### 5.5.1 Send Message from Arduino to Serial Monitor

🧰 Components:

- Arduino Uno board

- USB cable

- Arduino IDE

📝 Code:

void setup() {

Serial.begin(9600);              // Initialize serial communication

Serial.println("Hello, RP-GISHARI COLLEGE") ;// Send message once

}

void loop() {

Serial.println("Arduino is running...");

delay(1000); // Delay 1 second

}

### 5.5.2 Receive Data from Serial Monitor and Control LED

📝 Code 1:

String input;

int redLED = 10;

int yellowLED = 11;

int greenLED = 12;

int blueLED = 13;

void setup() {

Serial.begin(9600); // Start serial communication

pinMode(redLED, OUTPUT);

pinMode(yellowLED, OUTPUT);

pinMode(greenLED, OUTPUT);

pinMode(blueLED, OUTPUT);

Serial.println("Type RED ON / RED OFF / ALL OFF etc.");

}

void loop() {

if (Serial.available()) {

input = Serial.readStringUntil('\n');

input.trim(); // Remove any newline or spaces

if (input == "RED ON") {

digitalWrite(redLED, HIGH);    }

else if (input == "RED OFF") {

digitalWrite(redLED, LOW);    }

else if (input == "YELLOW ON") {

digitalWrite(yellowLED, HIGH);    }

else if (input == "YELLOW OFF") {

digitalWrite(yellowLED, LOW);    }

else if (input == "GREEN ON") {

digitalWrite(greenLED, HIGH);    }

else if (input == "GREEN OFF") {

digitalWrite(greenLED, LOW);    }

else if (input == "BLUE ON") {

digitalWrite(blueLED, HIGH);    }

else if (input == "BLUE OFF") {

digitalWrite(blueLED, LOW);    }

else if (input == "ALL OFF") {

digitalWrite(redLED, LOW);

digitalWrite(yellowLED, LOW);

digitalWrite(greenLED, LOW);

digitalWrite(blueLED, LOW);    }

else {

Serial.println("Invalid command.");    }  }

}

📝 Code 2:

🧰 Components:

- Arduino Uno

- 1 x LED

- 1 x 220Ω resistor

- Breadboard and jumper wires

💡 Circuit:

- LED anode → Pin 8

- LED cathode → 220Ω resistor → GND

void setup() {

pinMode(8, OUTPUT);

Serial.begin(9600);

Serial.println("Type ON or OFF:");

}

void loop() {

if (Serial.available()) {

String command = Serial.readStringUntil('\n'); // Read input

command.trim(); // Remove any extra space or newline

if (command == "ON") {

digitalWrite(8, HIGH);

Serial.println("LED is ON");

} else if (command == "OFF") {

digitalWrite(8, LOW);

Serial.println("LED is OFF");

} else {

Serial.println("Invalid Command");    }  }

}

## 5.6 Procedure

- Connect LEDs with 220Ω resistors to Arduino digital pins 10 to 13.

- Upload the provided code to the Arduino board.

- Open Serial Monitor from Arduino IDE (Baud Rate: 9600).

- Type commands like RED ON, YELLOW OFF, GREEN ON, BLUE OFF, or ALL OFF.

- Observe the LEDs turning ON or OFF based on typed command.

## 5.7 Observations

| Serial Command | LED Action |
| --- | --- |
| RED ON | Red LED turns ON |
| YELLOW ON | Yellow LED turns ON |
| GREEN OFF | Green LED turns OFF |
| BLUE ON | Blue LED turns ON |
| ALL OFF | All LEDs turn OFF |
| Invalid Command | No action; message displayed |

## 5.8 Review Questions

- What is serial communication in Arduino?

- Which function is used to read input from Serial Monitor?

- What is the purpose of Serial.begin()?

- How is data sent from Arduino to PC?

- Can serial commands be used to control motors or other actuators?

## 5.9 Conclusion

This experiment helped students understand the basics of Arduino serial communication and its practical use in controlling external devices like LEDs. This concept is useful in debugging, user input processing, and IoT device control through serial interface.

## 5.10 Practical exercises

1. Provide and simulate the arduino sketch to switch the LED ON if 'a' is typed in serial monitor and switch the LED off if 'B' is typed in serial monitor. Initialize serial communication at 19200 bits per second.

2. Develop a system by using serial monitor to switch the LED ON if the password ELT is entered and the Serial monitor displays LED ON. LED off will be switched OFF if the password ETT is entered in serial monitor and Serial monitor displays LED OFF.

# Experiment Six: Interfacing Analog and Digital Sensors to Control LEDs Using Arduino

## ✅ SENSOR 1: LDR (Light Dependent Resistor)

## 6.1 Interfacing LDR Sensor to Control LED

### 6.1.1 Objective

To interface an LDR sensor with Arduino and control an LED based on ambient light intensity.

### 6.1.2 Components Required

| Component | Quantity |
| --- | --- |
| Arduino Uno | 1 |
| LDR Sensor | 1 |
| 10kΩ Resistor | 1 |
| LED (Any color) | 1 |
| 220Ω Resistor | 1 |
| Breadboard | 1 |
| Jumper Wires | As needed |
| USB Cable | 1 |
| Laptop with IDE | 1 |

### 6.1.3 Circuit Diagram Description

- LDR forms a voltage divider with a 10kΩ resistor

- The middle point is connected to Arduino analog pin A0

- LED connected to digital pin (e.g., D8) with 220Ω resistor

### 6.1.4 Arduino Code

int ldrPin = A0;

int ledPin = 8;

int threshold = 500;

void setup() {

pinMode(ledPin, OUTPUT);

Serial.begin(9600);

}

void loop() {

int ldrValue = analogRead(ldrPin);

Serial.println(ldrValue);

if (ldrValue < threshold) {

digitalWrite(ledPin, HIGH); // Turn ON LED when dark

}

else {

digitalWrite(ledPin, LOW);  // Turn OFF LED in bright light

}

delay(500);

}

### 6.1.5 Procedure

- Connect the LDR and resistor in a voltage divider.

- Connect middle point to A0, and LED to pin D8.

- Upload the code and open Serial Monitor.

- Observe LDR values change with light.

- LED turns ON when light is low, OFF when light is high.

### 6.1.6 Observations

| LDR Value | LED Status |
| --- | --- |
| < 500 | ON |
| ≥ 500 | OFF |

### 6.1.7 Review  Questions

- What is an LDR and how does it work?

- Why use a voltage divider?

- Can LDR detect darkness or brightness?

- What is the analog input range of Arduino?

- What factors affect LDR sensitivity?

### 6.1.8 Practical exercises

- As we all know, there are places where the lamps are turned ON based on amount of light from daylight to night. Conduct an experiment with arduino and appropriate sensor to turn ON and OFF according to the amount of light detected by this sensor where the light of lamp varies with the amount of light detected. Assume that the minimum value from the sensor to turn ON the lamps is 400.

## ✅ SENSOR 2: LM35 (Temperature Sensor)

## 6.2 Interfacing LM35 Sensor to Control LED

### 6.2.1 Objective

To control an LED based on temperature measured by the LM35 sensor using Arduino.

### 6.2.2 Components Required

| Component | Quantity |
| --- | --- |
| Arduino Uno | 1 |
| LM35 Sensor | 1 |
| LED | 1 |
| 220Ω Resistor | 1 |
| Breadboard | 1 |
| Jumper Wires | As needed |
| USB Cable | 1 |
| Laptop with IDE | 1 |

### 6.2.3 Circuit Diagram Description

- LM35 output connected to A0

- LED connected to digital pin (e.g., D13)

- LM35 is powered from 5V and GND

### 6.2.4 Arduino Code

int lm35Pin = A0;

int ledPin = 13;

float temp;

void setup() {

pinMode(ledPin, OUTPUT);

Serial.begin(9600);

}

void loop() {

int sensorValue = analogRead(lm35Pin);

temp = (sensorValue * 5.0 * 100.0) / 1024.0;

Serial.print("Temperature: ");

Serial.println(temp);

if (temp > 30.0) {

digitalWrite(ledPin, HIGH); // Turn on LED if temp > 30°C

}

else {

digitalWrite(ledPin, LOW);  }

delay(1000);}

### 6.2.5 Procedure

- Connect LM35 as per circuit.

- Upload code and monitor temperature on Serial Monitor.

- LED turns ON above 30°C and OFF otherwise.

### 6.2.6 Observations

| Temperature | LED Status |
| --- | --- |
| > 30°C | ON |
| ≤ 30°C | OFF |

### 6.2.7 Review Questions

- What is the output scale of LM35?

- Why is analogRead used?

- Can LM35 be used outdoors?

- How do you convert analog to temperature?

- What is the resolution of analog pins?

## ✅ SENSOR 3: Potentiometer (Analog Input)

## 6.3 Interfacing Potentiometer with Arduino to Control LED

### 6.3.1 Objective

To interface a potentiometer with Arduino and control LED brightness using analog input (PWM).

### 6.3.2 Components Required

| Component | Quantity |
| --- | --- |
| Arduino Uno | 1 |
| Potentiometer | 1 |
| LED | 1 |
| 220Ω Resistor | 1 |
| Breadboard | 1 |
| Jumper Wires | As needed |
| USB Cable | 1 |
| Laptop with IDE | 1 |

### 6.3.3 Circuit Diagram Description

- Potentiometer middle pin → A0

- One outer pin to 5V, other to GND

- LED on PWM pin (e.g., D5) with resistor

### 6. 3.4 Arduino Code

int potPin = A0;

int ledPin = 5;//pwm pin

int potValue;

void setup() {

pinMode(ledPin, OUTPUT);

Serial.begin(9600);

}

void loop() {

potValue = analogRead(potPin);

int brightness = map(potValue, 0, 1023, 0, 255);

analogWrite(ledPin, brightness);

Serial.println(brightness);

delay(100);

}

### 6.3.5 Procedure

- Wire the potentiometer and LED as per diagram.

- Upload code and open Serial Monitor.

- Rotate the knob and observe LED brightness changes.

### 6.3.6 Observations

| Potentiometer Value | LED Brightness (PWM) |
| --- | --- |
| 0 - 1023 | 0 - 255 |

### 6.3.7 Review Questions

- What is the purpose of a potentiometer?

- What is PWM and where is it used?

- Why do we use map() function?

- Which pins support PWM on Arduino Uno?

- What is analogRead() range?

### 6.3.8 Practical exercises

1. Conduct a project with Arduino to control a brightness of LED with potentiometer and using LCD and Serial Monitor to indicate the value of potentiometer that corresponding with LED brightness.

## ✅ SENSOR 4: DHT11 (Temperature & Humidity Sensor)

## 6.4 Interfacing DHT11 Sensor with Arduino to Control LED

### 6.4.1 Objective

To interface a DHT11 temperature & humidity sensor with Arduino and turn ON an LED if temperature exceeds a certain level.

### 6.4.2 Components Required

| Component | Quantity |
| --- | --- |
| Arduino Uno | 1 |
| DHT11 Sensor | 1 |
| LED | 1 |
| 220Ω Resistor | 1 |
| Breadboard | 1 |
| Jumper Wires | As needed |
| USB Cable | 1 |
| Laptop | 1 |

### 6.4.3 Circuit Diagram Description

- Connect DHT11 output to pin D2

- Power it from 5V and GND

- LED connected to D8 with resistor

### 6. 4.4 Arduino Code

#include <DHT.h>

#define DHTPIN 2

#define DHTTYPE DHT11

DHT dht (DHTPIN, DHTTYPE);

int ledPin = 8;

void setup() {

Serial.begin(9600);

dht.begin();

pinMode(ledPin, OUTPUT);

}

void loop() {

float temp = dht.readTemperature();

Serial.println(temp);

if (temp > 28) {

digitalWrite(ledPin, HIGH); // LED ON

}

else {

digitalWrite(ledPin, LOW); // LED OFF

}

delay(2000);

}

### 6.4.5 Procedure

- Connect DHT11 as per the schematic.

- Upload the code.

- Open Serial Monitor to read temperature.

- Observe LED turns ON if temperature > 28°C.

### 6.4.6 Observations

| Temperature | LED Status |
| --- | --- |
| > 28°C | ON |
| ≤ 28°C | OFF |

### 6.4.7 Review Questions

- What values does DHT11 return?

- Why do we use a library for DHT11?

- What is the refresh rate of DHT11?

- Can DHT11 detect humidity alone?

- What are common applications?

### 6.4.8 Practical exercises

1. Conduct a project with Arduino using the temperature sensor, LEDs ,LCD and the Serial monitor. The program should continuously read the temperature value from the temperature sensor and if the temperature value is:

- Less than or equal to 28oC then display the message “Very Cold” on the screens and set the light color for the LED to Blue.

- Greater than 28 and less than or equal to 30 oC then display the message “Cold” on the screens and set the light color for the LED to Green.

- Greater than 30 and less than or equal to 35oC then display the message “Warm” on the screens and set the light color for the LED to Yellow.

- Greater than 35 oC then display the message “Very Hot” on the screen, set the light color for the LED to Red .

Note: The value of temperature should be also displayed in LCD and in Serial Monitor as for example 35oC.

- As Electronician, you are request to design and implement a system to control a DC motor of Fan by either using temperature or computer, if the word “ON“ is typed in serial monitor or temperature value is greater than 38.2oC, the DC motor of Fan will turn on automatically with a flashing red LED and if the word “OFF” is typed or temperature value is less than or equal 36.7oC, the fan will turn off automatically. Display the value of the temperature in Celsius in serial monitor and LCD.

## ✅ SENSOR 5: DHT22 (Digital Temperature & Humidity Sensor – High Accuracy)

## 6.5 Interfacing DHT22 Sensor with Arduino to Control LED

### 6.5.1 Arduino Code

#include <DHT.h>

#define DHTPIN 2

#define DHTTYPE DHT22

DHT dht(DHTPIN, DHTTYPE);

int ledPin = 8;

void setup() {

Serial.begin(9600);

dht.begin();

pinMode(ledPin, OUTPUT);

}

void loop() {

float temperature = dht.readTemperature();

float humidity = dht.readHumidity();

Serial.print("Temp: "); Serial.print(temperature);

Serial.print(" °C, Hum: "); Serial.println(humidity);

if (temperature > 30 || humidity > 70) {

digitalWrite(ledPin, HIGH);  // LED ON

} else {

digitalWrite(ledPin, LOW);   // LED OFF

}

delay(2000);}

### 6.5.2 Observations

| Temp (°C) / Humidity (%) | LED Status |
| --- | --- |
| Temp > 30 or Hum > 70 | ON |
| Otherwise | OFF |

### 6.5.3 Review Questions

- How is DHT22 better than DHT11?

- What is the response time of DHT22?

- Can you use both temp and humidity for control?

- What’s the accuracy of DHT22?

- Why do we use delay in the loop?

## ✅ SENSOR 6: Ultrasonic Sensor (HC-SR04)

## 6.6 Interfacing Ultrasonic Sensor to Control LED

### 6.6.1 Objective

To control an LED based on the distance measured by the HC-SR04 ultrasonic sensor.

### 6.6.2 Components Required

| Component | Quantity |
| --- | --- |
| Arduino Uno | 1 |
| HC-SR04 Sensor | 1 |
| LED | 1 |
| 220Ω Resistor | 1 |
| Breadboard | 1 |
| Jumper Wires | As needed |
| USB Cable | 1 |
| Laptop | 1 |

### 6.6.3 Circuit Diagram Description

- Trigger pin → D9, Echo pin → D8

- LED → D7 with 220Ω resistor

### 6.6.4 Arduino Code

#include <Wire.h>

#include <LiquidCrystal_I2C.h>

// Set the LCD address to 0x27 or 0x3F (depending on your module)

LiquidCrystal_I2C lcd(0x27, 16, 2);

const int trigPin = 9;

const int echoPin = 8;

const int ledPin = 7;

void setup() {

lcd.init();      // Initialize LCD

lcd.backlight();  // Turn on backlight

pinMode(trigPin, OUTPUT);

pinMode(echoPin, INPUT);

pinMode(ledPin, OUTPUT);

Serial.begin(9600);

}

void loop() {

digitalWrite(trigPin, LOW);

delayMicroseconds(2);

digitalWrite(trigPin, HIGH);

delayMicroseconds(10);

digitalWrite(trigPin, LOW);

long duration = pulseIn(echoPin, HIGH);

int distance = duration * 0.034 / 2;

Serial.println(distance);

lcd.setCursor(0, 0);

lcd.print("Dist:");

lcd.print(distance);

lcd.print(" cm");

if (distance < 10) {

digitalWrite(ledPin, HIGH); // Object too close

} else {

digitalWrite(ledPin, LOW);

}

delay(500);

}

### 6.6.5 Procedure

- Connect HC-SR04 to Arduino as per circuit diagram.

- Upload the code.

- Monitor distance in Serial Monitor.

- Move object close (<10cm) to turn LED ON.

### 6.6.6 Observations

| Distance (cm) | LED Status |
| --- | --- |
| < 10 | ON |
| ≥ 10 | OFF |

### 6.6.7 Review Questions

- How does HC-SR04 work?

- What is pulseIn()?

- How is distance calculated from time?

- What is the range of the sensor?

- Can it detect transparent objects?

### 6.6.8 Practical exercise

1. Conduct a project with Arduino to indicate the range of detected object using appropriate sensor and LEDs and using LCD and Serial Monitor to display the value of range. Blue LED indicates that the detected object is above 70cm, green LED indicates that the detected object is between 70cm and 50cm, otherwise Red LED indicates that the detected object is not located on the mentioned range.

2. As an electronic technician, you are requested to implement a project of recoding and displaying with LCD distance value from sensor and use that distance to control a red LED by flashing and Buzzer ON if object is detected at a distance which is less than or equal to 100cm. Display also” OBJECT DETECTED” or “NO OBJECT DETECTED”.

## ✅ SENSOR 7: Infrared (IR) Obstacle Sensor

## 6.7 Interfacing IR Obstacle Sensor to Control LED

### 6.7.1 Objective

To control an LED when an object is detected using IR obstacle sensor.

### 6.7.2 Components Required

| Component | Quantity |
| --- | --- |
| Arduino Uno | 1 |
| IR Sensor | 1 |
| LED | 1 |
| 220Ω Resistor | 1 |
| Breadboard | 1 |
| Jumper Wires | As needed |
| USB Cable | 1 |
| Laptop | 1 |

### 6.7.3 Circuit Diagram Description

- IR output → D2

- LED → D3 with 220Ω resistor

### 6.7.4 Arduino Code

#include <Wire.h>

#include <LiquidCrystal_I2C.h>

LiquidCrystal_I2C lcd(0x27, 16, 2);

int irPin = 2;

int ledPin = 3;

void setup() {

pinMode(irPin, INPUT);

pinMode(ledPin, OUTPUT);

Serial.begin(9600);

lcd.init();      // Initialize LCD

lcd. Backlight (); // Turn on backlight

}

void loop() {

int state = digitalRead(irPin);

Serial.println(state);

if (state == LOW) {

digitalWrite (ledPin, HIGH); // Object detected

lcd.setCursor(0, 0);

lcd.print ("Object detected");

}

else {

digitalWrite (ledPin, LOW); // No object

lcd.setCursor(0, 0);

lcd.print ("No Object detected");

}

delay (100);

}

### 6.7.5 Procedure

- Connect IR sensor and LED.

- Upload the code.

- Bring object close to IR sensor → LED turns ON.

### 6.7.6 Observations

| IR Output | LED Status |
| --- | --- |
| LOW | ON |
| HIGH | OFF |

### 6.7.7 Review Questions

- What is the principle of IR sensors?

- How does reflectivity affect detection?

- Can IR detect in sunlight?

- Why is output LOW when object is nearby?

- Difference between active and passive sensors?

### 6.7.8 Practical exercises

As an electronic technician, you are requested to provide a system to count persons through the stadium gate with a flashing green LED. An LCD prints the number of persons and prints also “PERSON IS ENTERED!!!!!” if there is a person and clear LCD if no person entering through the stadium gate.

## ✅ SENSOR 8: PIR Motion Sensor

## 6.8. Interfacing PIR Motion Sensor to Control LED

### 6.8.1 Objective

To detect motion using a PIR sensor and control an LED accordingly.

### 6.8.2 Components Required

| Component | Quantity |
| --- | --- |
| Arduino Uno | 1 |
| PIR Sensor | 1 |
| LED | 1 |
| 220Ω Resistor | 1 |
| Breadboard | 1 |
| Jumper Wires | As needed |
| USB Cable | 1 |
| Laptop | 1 |

### 6.8.3 Circuit Diagram Description

- PIR output → D2

- LED → D7 via 220Ω resistor

### 6.8.4 Arduino Code

int pirPin = 2;

int ledPin = 7;

void setup() {

pinMode(pirPin, INPUT);

pinMode(ledPin, OUTPUT);

Serial.begin(9600);

}

void loop() {

int motion = digitalRead(pirPin);

Serial.println(motion);

if (motion == HIGH) {

digitalWrite(ledPin, HIGH); // Motion detected

} else {

digitalWrite(ledPin, LOW);  // No motion

}

delay(100);

}

### 6.8.5 Procedure

- Connect PIR sensor and LED.

- Upload the code and observe LED behavior.

- Move in front of sensor → LED turns ON.

### 6.8.6 Observations

| Motion Detected | LED Status |
| --- | --- |
| YES (HIGH) | ON |
| NO (LOW) | OFF |

### 6.8.7 Review Questions

- How does a PIR sensor work?

- What does passive mean in PIR?

- What is the detection range?

- Why is there delay/reset time?

### 6.7.8 Practical exercises

As an electronic technician, you are requested to provide a system to count persons through the stadium gate with a flashing green LED. An LCD prints the number of persons and prints also “PERSON IS ENTERED!!!!!” if there are  persons and clear LCD if no person entering through the stadium gate.

# Experiment Seven: Interfacing Arduino Keypad to Control LEDs

## 7.1 Objective

To design and implement an interface between a 4x4 or 4x3 keypad and Arduino Uno to control multiple LEDs by pressing specific keys.

## 7.2 Components Required

| Component | Quantity |
| --- | --- |
| Arduino Uno | 1 |
| 4x4 Keypad Module | 1 |
| LEDs (Red/Green/Yellow) | 4 |
| 220Ω Resistors | 4 |
| Breadboard | 1 |
| Jumper wires | As needed |
| USB cable | 1 |
| Laptop with Arduino IDE | 1 |

## 7.3 Circuit Diagram Description

- Connect the rows and columns of the keypad to Arduino digital pins.

- Connect each LED (with resistor) to a separate digital pin.

- The Arduino will detect key presses and switch LEDs ON/OFF accordingly.

🔌 Pin Connections

🔲 Keypad Pins → Arduino

| Keypad Pin | Arduino Digital Pin |
| --- | --- |
| Row 1 | D9 |
| Row 2 | D8 |
| Row 3 | D7 |
| Row 4 | D6 |
| Col 1 | D5 |
| Col 2 | D4 |
| Col 3 | D3 |

💡 LED Pins → Arduino

| LED Color | Arduino Pin | Resistor |
| --- | --- | --- |
| Red | D10 | 220Ω |
| Yellow | D11 | 220Ω |
| Green | D12 | 220Ω |
| Blue | D13 | 220Ω |

## 7.4 Arduino Code

#include <Keypad.h>

const byte ROWS = 4;

const byte COLS = 3;

char keys[ROWS][COLS] = {

{'1','2','3'},

{'4','5','6'},

{'7','8','9'},

{'*','0','#'}

};

byte rowPins[ROWS] = {9, 8, 7, 6}; // Connect to the row pinouts

byte colPins[COLS] = {5, 4, 3};    // Connect to the column pinouts

Keypad keypad = Keypad(makeKeymap(keys), rowPins, colPins, ROWS, COLS);

int led1 = 10;

int led2 = 11;

int led3 = 12;

int led4 = 13;

void setup(){

Serial.begin(9600);

pinMode(led1, OUTPUT);

pinMode(led2, OUTPUT);

pinMode(led3, OUTPUT);

pinMode(led4, OUTPUT);

}

void loop(){

char key = keypad.getKey();

if(key) {

Serial.println(key);

switch(key) {

case '1': digitalWrite(led1, HIGH); break;

case '2': digitalWrite(led2, HIGH); break;

case '3': digitalWrite(led3, HIGH); break;

case '4': digitalWrite(led4, HIGH); break;

case '5': digitalWrite(led1, LOW); break;

case '6': digitalWrite(led2, LOW); break;

case '7': digitalWrite(led3, LOW); break;

case '8': digitalWrite(led4, LOW); break;

case '0':

digitalWrite(led1, LOW);

digitalWrite(led2, LOW);

digitalWrite(led3, LOW);

digitalWrite(led4, LOW);

break;  }}}

## 7. 5 Procedure

- Connect the keypad to the Arduino according to the pin mapping.

- Connect each LED and resistor to digital output pins.

- Upload the code to the Arduino board.

- Open Serial Monitor to observe keypress values.

- Press keys 1 to 4 to turn ON LEDs.

- Press keys 5 to 8 to turn OFF corresponding LEDs.

- Press 0 to turn OFF all LEDs at once.

## 7.6 Observations

| Key Pressed | LED Action |
| --- | --- |
| '1' | Red ON |
| '2' | Yellow ON |
| '3' | Green ON |
| '4' | Blue ON |
| '5' to '8' | Corresponding OFF |
| '0' | All LEDs OFF |

## 7.7 Review Questions

- What is the role of a keypad in embedded systems?

- How does the Arduino detect which key is pressed?

- What are rows and columns in keypad interfacing?

- What is the use of the Keypad library in Arduino?

- Can you interface a keypad with other output devices besides LEDs?

## 7.8 Conclusion

This experiment demonstrates how to interface a keypad with Arduino and control external devices like LEDs. This concept is fundamental in designing embedded user interfaces such as door locks, access panels, or control systems.

## 7.9 Practical exercise

# Experiment Eight: Interfacing 16x2 LCD with Arduino to display messages and control LEDs

## 8.1 Objective

To use an LCD to display control options and switch ON/OFF an LED via input or internal conditions.

## 8.2 Components Required

| Component | Quantity |
| --- | --- |
| Arduino Uno | 1 |
| 16x2 LCD Display | 1 |
| Potentiometer (10k) | 1 |
| LEDs (Red/Green) | 2 |
| 220Ω Resistors | 2 |
| Breadboard | 1 |
| Jumper wires | As needed |

## 8.3 Circuit Diagram Description

- LCD RS → D12, E → D11, Data4 to Data7 → D5-D2

- Connect potentiometer for contrast adjustment

- LED1 → D8, LED2 → D9

## 8.4 Arduino Code

#include <LiquidCrystal.h>

LiquidCrystal lcd(12, 11, 5, 4, 3, 2);

int led1 = 8;

int led2 = 9;

void setup() {

lcd. begin (16, 2);

pinMode (led1, OUTPUT);

pinMode (led2, OUTPUT);

lcd.print ("LEDs ON");

digitalWrite (led1, HIGH);

digitalWrite (led2, HIGH);

delay (2000);

lcd. clear ();

lcd.print("LEDs OFF");

digitalWrite(led1, LOW);

digitalWrite(led2, LOW);

}

void loop () { }

## 8.5 Procedure

- Build circuit and connect LCD.

- Connect LEDs with resistors.

- Upload code.

- Observe LCD messages and LED control.

## 8. 6 Observations

| Message | LED Status |
| --- | --- |
| "LEDs ON" | Both ON |
| "LEDs OFF" | Both OFF |

## 8.7 Review Questions

- How many pins does a 16x2 LCD use?

- How is contrast adjusted?

- Can LCDs take input?

# Experiment Nine : Interfacing Arduino with I2C LCD Display

## 9.1 Objective

To interface a 16x2 I2C LCD with Arduino Uno and display text or sensor values using I2C communication.

## 9.2 Components Required

| Component | Quantity |
| --- | --- |
| Arduino Uno | 1 |
| 16x2 LCD with I2C module | 1 |
| Breadboard (optional) | 1 |
| Jumper wires | As needed |
| USB cable | 1 |
| Laptop with Arduino IDE | 1 |

## 9.3 Circuit Diagram Description

- The I2C LCD uses only 2 pins: SDA and SCL

- SDA connects to Arduino A4, and SCL connects to A5

- The LCD is powered using 5V and GND

📦 Library Required
Install LiquidCrystal_I2C library from the Arduino Library Manager.

## 9.4 Arduino Code

#include <Wire.h>

#include <LiquidCrystal_I2C.h>

// Set the LCD address to 0x27 or 0x3F (depending on your module)

LiquidCrystal_I2C lcd(0x27, 16, 2);

void setup() {

lcd.begin();      // Initialize LCD

lcd.backlight();  // Turn on backlight

lcd.setCursor(0, 0);

lcd.print("Hello TVET!");

lcd.setCursor(0, 1);

lcd.print("LCD I2C Ready!");

}

void loop() {

// Nothing to do here

}

📌 Note: If nothing appears on LCD, try changing 0x27 to 0x3F.
📌 You can also run the I2C Scanner sketch to find the correct address.

## 9.5 Procedure

- Connect the I2C LCD to Arduino as per the pin mapping.

- Install the required library (LiquidCrystal_I2C) from Library Manager.

- Upload the code to the Arduino Uno.

- Observe the text “Hello TVET!” and “LCD I2C Ready!” displayed on the screen.

- Adjust contrast if needed using the potentiometer on the I2C module.

## 9.6 Observations

| Operation | LCD Display |
| --- | --- |
| Code uploaded | "Hello TVET!" (Row 1) |
|  | "LCD I2C Ready!" (Row 2) |

## 9.7 Review  Questions

- What is the role of I2C protocol in LCD interfacing?

- What is the difference between parallel and I2C LCDs?

- What is the I2C address and how do you find it?

- Why are only two pins used in I2C communication?

- Can multiple I2C devices be connected to Arduino? How?

## 9.8 Conclusion

This experiment demonstrated the use of I2C communication to interface a 16x2 LCD with Arduino using only two data pins, simplifying wiring and saving digital I/O resources.

# Experiment Ten: Control LED by servo position using Arduino

## 10.1 Objective

To turn ON an LED when servo reaches specific angles

## 10.2 Components Required

| Component | Quantity |
| --- | --- |
| Arduino Uno | 1 |
| Servo Motor (SG90) | 1 |
| LED | 1 |
| Resistor (220Ω) | 1 |
| Breadboard, Jumpers | As needed |

## 10.3 Pin Connections

- Servo signal → D9

- LED → D8

## 10.4 Arduino Code

#include <Servo.h>

Servo myservo;

int led = 8;

void setup() {

myservo.attach(9);

pinMode(led, OUTPUT);

}

void loop () {

myservo. write (0);

digitalWrite (led, LOW);

delay (1000);

myservo. write (90);

digitalWrite (led, HIGH);

delay (1000);

}

## 10.5 Practical exercise

Conduct a project with Arduino, servo motor and temperature sensor where the servo motor rotate to angle between 20 to 220 degrees with respect to the value from temperature sensor from DHT sensor.

# Experiment Eleven: Interfacing Bluetooth Module (HC-05) with Arduino to Control LEDs

## 11.1 Objective

To design and implement wireless control of LEDs using a smartphone and the HC-05 Bluetooth module.

## 11.2 Components Required

| Component | Quantity |
| --- | --- |
| Arduino Uno | 1 |
| Bluetooth Module HC-05 | 1 |
| LEDs (Red/Green/Blue) | 3 |
| 220Ω Resistors | 3 |
| Breadboard | 1 |
| Jumper wires | As needed |
| Android phone with Bluetooth terminal | 1 |

## 11.3 Circuit Diagram Description

- HC-05 VCC → Arduino 5V

- HC-05 GND → Arduino GND

- HC-05 TX → Arduino RX (Pin 0)

- HC-05 RX → Arduino TX (Pin 1) (Use voltage divider or level shifter)

- LEDs connected to Pins D10, D11, and D12 via 220Ω resistors

## 11.4 Arduino Code

char data = 0;

void setup() {

Serial.begin(9600);

pinMode(10, OUTPUT);

pinMode(11, OUTPUT);

pinMode(12, OUTPUT);

}

void loop() {

if(Serial.available()) {

data = Serial.read();

if(data == 'R') {

digitalWrite(10, HIGH); }

if(data == 'r') {

digitalWrite(10, LOW); }

if(data == 'G') {

digitalWrite(11, HIGH); }

if(data == 'g') {

digitalWrite(11, LOW); }

if(data == 'B') {

digitalWrite(12, HIGH); }

if(data == 'b') {

digitalWrite(12, LOW); }

}}

## 11.5 Procedure

- Connect the HC-05 to Arduino as per the diagram.

- Connect LEDs to digital pins.

- Upload the code.

- Pair HC-05 with your phone (default password: 1234/0000).

- Use a Bluetooth terminal app to send characters: R, G, B to turn ON and r, g, b to turn OFF.

## 11.6 Observations

| Key Sent | LED Action |
| --- | --- |
| R | Red ON |
| r | Red OFF |
| G | Green ON |
| g | Green OFF |
| B | Blue ON |
| b | Blue OFF |

## 11.7 Review Questions

- What is the function of HC-05?

- Why do we use Serial communication in this experiment?

- Can Bluetooth be used to read sensor data too?

- How is HC-05 different from HC-06?

- How can you secure Bluetooth communication?

## 11.8 Conclusion

Bluetooth interfacing with Arduino enables basic wireless control and can be extended to home automation and robotics.

## 11.9 Practical exercise

1. A Bluetooth controlled smart lights using arduino uno/mega is an innovative experiment that combines the power of Bluetooth technology and the flexibility of arduino microcontrollers to create a smart lighting system  that can be controlled wirelessly using a smartphone or tablet. With this experiment, you can turn on/off your lights with just a few taps on your mobile device. Conduct an experiment to control five LEDs(green, blue, yellow ,white and red) in the following ways:

- Green LED blinks six times with 0.2second ON and 0.2second OFF time.

- Blue LED blinks five times with 0.4second ON and 0.1second OFF time.

- Yellow LED four nine times with 0.4second ON and 0.3second OFF time.

- Red LED blinks three times with 0.3second ON and 0.3second OFF time.

- White LED blinks two times with 0.5 second ON time and 0.5 second OFF time.

# Experiment Twelve: Interfacing Stepper motor with Arduino

## 12.1 Objective

To interface a stepper motor with an Arduino Uno and control its direction and speed of rotation using digital output signals.

## 12.2 Components Required

| Component | Quantity |
| --- | --- |
| Arduino Uno | 1 |
| Stepper Motor (28BYJ-48) | 1 |
| ULN2003 Driver Module | 1 |
| External Power Supply (5V–12V) | 1 |
| Breadboard | 1 |
| Jumper Wires | As required |

## 12.3 Circuit Diagram Description

Stepper Motor and Driver Connections

- Stepper motor connected to ULN2003 driver board

- ULN2003 IN1 → Arduino D8

- ULN2003 IN2 → Arduino D9

- ULN2003 IN3 → Arduino D10

- ULN2003 IN4 → Arduino D11

- ULN2003 VCC → External 5V

- ULN2003 GND → Arduino GND

(Ensure common ground between Arduino and external power supply)

## 12.4 Arduino Code

Code1:

#include <Stepper.h>

const int stepsPerRevolution = 2048;

const int rpm = 5;

Stepper stepper1 = Stepper(stepsPerRevolution, 8, 10, 9, 11);

void setup() {

stepper1.setSpeed(rpm);

}

void loop() {

stepper1.step(stepsPerRevolution);

delay(100);

stepper1.step(-stepsPerRevolution);

delay(100);

}

Code2:   Stepper motor control using serial monitor

char command;

int pins[] = {8, 9, 10, 11};

int seq[4][4] = {

{1,0,0,0},

{0,1,0,0},

{0,0,1,0},

{0,0,0,1}

};

void setup() {

Serial.begin(9600);

for(int i=0;i<4;i++) pinMode(pins[i], OUTPUT);

Serial.println("Enter C for Clockwise, A for Anticlockwise");

}

void loop() {

if (Serial.available()) {

command = Serial.read();

if (command == 'C') rotateCW();

if (command == 'A') rotateCCW();

}

}

void rotateCW() {

for(int i=0;i<50;i++)

for(int j=0;j<4;j++) stepMotor(j);

}

void rotateCCW() {

for(int i=0;i<50;i++)

for(int j=3;j>=0;j--) stepMotor(j);

}

void stepMotor(int step) {

for(int i=0;i<4;i++)

digitalWrite(pins[i], seq[step][i]);

delay(10);

}

Code3: Stepper motor control using keypad

#include <Keypad.h>

const byte rows = 4, cols = 4;

char keys[rows][cols] = {

{'1','2','3','A'},

{'4','5','6','B'},

{'7','8','9','C'},

{'*','0','#','D'}

};

byte rowPins[rows] = {2,3,4,5};

byte colPins[cols] = {6,7,12,13};

Keypad keypad = Keypad(makeKeymap(keys), rowPins, colPins, rows, cols);

int motorPins[] = {8,9,10,11};

int seq[4][4] = {

{1,0,0,0},

{0,1,0,0},

{0,0,1,0},

{0,0,0,1}

};

void setup() {

for(int i=0;i<4;i++) pinMode(motorPins[i], OUTPUT);

}

void loop() {

char key = keypad.getKey();

if(key == '1') rotateCW();

if(key == '2') rotateCCW();

}

void rotateCW() {

for(int i=0;i<50;i++)

for(int j=0;j<4;j++) step(j);

}

void rotateCCW() {

for(int i=0;i<50;i++)

for(int j=3;j>=0;j--) step(j);

}

void step(int s) {

for(int i=0;i<4;i++)

digitalWrite(motorPins[i], seq[s][i]);

delay(10);

}

## 12.5 Procedure

- Connect the stepper motor to the ULN2003 driver.

- Interface the driver module with Arduino as per the circuit description.

- Provide external power to the motor.

- Connect Arduino to the computer.

- Upload the program to the Arduino.

- Observe the stepper motor rotation.

- Note direction change and speed variations.

## 12.6 Observations

| Action | Observation |
| --- | --- |
| Clockwise stepping | Motor rotates smoothly |
| Anticlockwise stepping | Direction reverses |
| Increase delay | Motor speed decreases |
| Decrease delay | Motor speed increases |

## 12.7 Review Questions

- What is a stepper motor?

- Why is a driver circuit required for a stepper motor?

- What is the function of ULN2003?

- Differentiate between stepper motor and DC motor.

- How is motor speed controlled in this experiment?

- How can you secure Bluetooth communication?

## 12.8 Conclusion

In this experiment, a stepper motor was successfully interfaced with an Arduino Uno using a ULN2003 driver. Precise control of motor direction and speed was achieved through programmed stepping sequences, demonstrating the suitability of stepper motors for applications requiring accurate positioning such as robotics and CNC systems.

# Experiment Thirteen: Interfacing Seven Segment Display with Arduino to Indicate LED Control

## 13.1 Objective

To control LEDs via Arduino and display their status using a seven-segment display.

## 13.2 Components Required

| Component | Quantity |
| --- | --- |
| Arduino Uno | 1 |
| Common Cathode 7-Segment Display | 1 |
| LEDs (Red/Green) | 2 |
| 220Ω Resistors | 5 |
| Breadboard | 1 |
| Jumper wires | As needed |

## 13.3 Circuit Diagram Description

- Connect segments (a-g) of 7-segment to D2–D8

- Connect common cathode to GND

- Connect LEDs to D10 and D11

- Use code to show “1” for Red ON, “2” for Green ON, “0” for both OFF

https://circuitdigest.com/microcontroller-projects/interfacing-seven-segment-display-with-Arduino

## 13.4 Arduino Code

Code 1:

int seg[] = {2, 3, 4, 5, 6, 7, 8};

byte digits [3][7] = {

{1,1,1,1,1,1,0}, // 0

{0,1,1,0,0,0,0}, // 1

{1,1,0,1,1,0,1}  // 2

};

void displayDigit(byte num) {

for(int i=0;i<7;i++)

digitalWrite(seg[i],

digits[num][i]);

}

void setup() {

for(int i=0;i<7;i++) {

pinMode(seg[i], OUTPUT);

pinMode(10, OUTPUT); // Red

pinMode(11, OUTPUT); // Green

}}

void loop() {

digitalWrite(10, HIGH);

displayDigit(1);

delay(2000);

digitalWrite(10, LOW);

digitalWrite(11, HIGH);

displayDigit(2);

delay(2000);

digitalWrite(11, LOW);

displayDigit(0);

delay(2000);

}

Code 2:

4-digit Seven segment display (common CATHODE)

4-digit Seven segment display (common Anode)

#include "SevSeg.h"

SevSeg sevseg; //Instantiate a seven segment controller object

void setup() {

byte numDigits = 4;

byte digitPins[] = {2, 3, 4, 5};

byte segmentPins[] = {6, 7, 8, 9, 10, 11, 12, 13};

bool resistorsOnSegments = false; // 'false' means resistors are on digit pins

byte hardwareConfig = COMMON_ANODE;

bool updateWithDelays = false; // Default 'false' is Recommended

bool leadingZeros = false; // Use 'true' if you'd like to keep the leading zeros

bool disableDecPoint = false; // Use 'true' if your decimal point doesn't exist or isn't connected

sevseg.begin(hardwareConfig, numDigits, digitPins, segmentPins, resistorsOnSegments,

updateWithDelays, leadingZeros, disableDecPoint);

sevseg.setBrightness(90);

}

void loop() {

static unsigned long timer = millis();

static int deciSeconds = 0;

if (millis() - timer >= 100) {

timer += 100;

deciSeconds++; // 100 milliSeconds is equal to 1 deciSecond

if (deciSeconds == 10000) { // Reset to 0 after counting for 1000 seconds.

deciSeconds=0;

}

sevseg.setNumber(deciSeconds, 1);

}

sevseg.refreshDisplay(); // Must run repeatedly

}

Code 3: Arduino Program (Common Cathode – Display 1234)

#include "SevSeg.h"

SevSeg sevseg;

void setup() {

byte numDigits = 4;

byte digitPins[] = {2, 3, 4, 5};

byte segmentPins[] = {6, 7, 8, 9, 10, 11, 12, 13};

bool resistorsOnSegments = 0;

sevseg.begin(COMMON_ANODE, numDigits, digitPins, segmentPins, resistorsOnSegments);

sevseg.setBrightness(60);

}

void loop() {

sevseg.setChars("1234"); // set the characters/numbers to display.

sevseg.refreshDisplay();}

code 4:

int segPins[] = {6,7,8,9,10,11,12};   // a b c d e f g

int digitPins[] = {2,3,4,5};  // D1 D2 D3 D4

// Segment patterns for common anode (LOW = ON)

byte numbers[10][7] = {

{0,0,0,0,0,0,1}, // 0

{1,0,0,1,1,1,1}, // 1

{0,0,1,0,0,1,0}, // 2

{0,0,0,0,1,1,0}, // 3

{1,0,0,1,1,0,0}, // 4

{0,1,0,0,1,0,0}, // 5

{0,1,0,0,0,0,0}, // 6

{0,0,0,1,1,1,1}, // 7

{0,0,0,0,0,0,0}, // 8

{0,0,0,0,1,0,0}  // 9

};

void setup() {

for(int i=0;i<7;i++)

pinMode(segPins[i], OUTPUT);

for(int i=0;i<4;i++)

pinMode(digitPins[i], OUTPUT);

}

void loop() {

for(int count = 0; count <= 205; count++) {

unsigned long startTime = millis();

while(millis() - startTime < 100) { // display each number for 1 second

displayNumber(count);

}

}

}

// -------- Display Functions --------

void displayNumber(int num) {

int d[4];

d[0] = num / 1000;

d[1] = (num / 100) % 10;

d[2] = (num / 10) % 10;

d[3] = num % 10;

for(int i=0;i<4;i++) {

digitalWrite(digitPins[i], HIGH); // enable digit (CA)

showDigit(d[i]);

delay(5);

digitalWrite(digitPins[i], LOW);  // disable digit

}

}

void showDigit(int n) {

for(int i=0;i<7;i++) {

digitalWrite(segPins[i], numbers[n][i]);

}

}

## 13.5 Procedure

- Wire the seven segment and LEDs as per instructions.

- Upload the code.

- Observe LED ON/OFF state with segment display changing accordingly.

## 13.6 Observations

| Display | LED State |
| --- | --- |
| 1 | Red ON |
| 2 | Green ON |
| 0 | All LEDs OFF |

## 13.7 Review Questions

- What is the difference between common cathode and anode displays?

- How are digits displayed using code?

- Can we connect multiple 7-segments?

- How do you display characters like A, b, C?

## 13.8 Practical exercises

Q1.

Q2. Count from 200 to 0

# Experiment Fourteen: Interfacing LED Matrix/Dot matrix with arduino

## 14.1 Objective

To interface an 8×8 LED dot matrix display with an Arduino and display alphabetical letters from A to I sequentially.

## 14.2 Components Required

- Arduino Uno / Nano

- 8×8 LED Matrix with MAX7219 driver

- Jumper wires

- USB cable

## 14.3 Circuit Diagram Description

| MAX7219 Pin | Arduino Uno |
| --- | --- |
| VCC | 5V |
| GND | GND |
| DIN | D11 |
| CS | D10 |
| CLK | D13 |

Install Required Library

In Arduino IDE:

- Go to Sketch → Include Library → Manage Libraries

- Search for “LedControl”

- Install LedControl by Eberhard Fahle

## 14.4 Arduino Code

Code 1:

#include <LedControl.h>

// DIN, CLK, CS

LedControl lc = LedControl(11, 13, 10, 1);

// Alphabet patterns (8x8)

byte letters[10][8] = {

{0x18,0x24,0x42,0x7E,0x42,0x42,0x42,0x00}, // A

{0x7C,0x42,0x42,0x7C,0x42,0x42,0x7C,0x00}, // B

{0x3C,0x42,0x40,0x40,0x40,0x42,0x3C,0x00}, // C

{0x78,0x44,0x42,0x42,0x42,0x44,0x78,0x00}, // D

{0x7E,0x40,0x40,0x7C,0x40,0x40,0x7E,0x00}, // E

{0x7E,0x40,0x40,0x7C,0x40,0x40,0x40,0x00}, // F

{0x3C,0x42,0x40,0x4E,0x42,0x42,0x3C,0x00}, // G

{0x42,0x42,0x42,0x7E,0x42,0x42,0x42,0x00}, // H

{0x3C,0x18,0x18,0x18,0x18,0x18,0x3C,0x00}, // I

{0x1E,0x04,0x04,0x04,0x44,0x44,0x38,0x00} // J

};

void setup() {

lc.shutdown(0, false);

lc.setIntensity(0, 8);

lc.clearDisplay(0);

}

void loop() {

for (int i = 0; i < 10; i++) {

for (int row = 0; row < 8; row++) {

lc.setRow(0, row, letters[i][row]);

}

delay (800);  // display time per letter

lc.clearDisplay(0);

}

}

## 14.5 Observations

| Letter Displayed | Observation |
| --- | --- |
| A | Letter A is clearly visible on the matrix |
| B | Letter B is displayed after a short delay |
| C | Letter C appears correctly |
| D | Letter D is displayed |
| E | Letter E is displayed |
| F | Letter F is displayed |
| G | Letter G is displayed |
| H | Letter H is displayed |
| I | Letter I is displayed |
| Sequence | Letters appear in correct alphabetical order |

## 14.6 Review Questions

- What is an LED dot matrix?

- Why is the MAX7219 driver IC used?

- What is multiplexing in LED displays?

- How many rows and columns are present in an 8×8 LED matrix?

- How is an alphabet character represented in an LED matrix?

- What is the function of the delay () function in this experiment?

- How can brightness of the LED matrix be controlled?

## 14.7 Conclusion

Interfacing an LED dot matrix with Arduino using the MAX7219 driver is simple and efficient. The experiment demonstrates how bitmap patterns can be used to display characters. Sequential display of alphabets helps in understanding display control, timing, and embedded programming concepts.

## 14.8 Practical exercises

1. Perform the following:

- Modify the program to display alphabets from A to Z.

- Change the delay time to control the display speed.

- Display only vowel letters on the LED matrix.

- Create a scrolling text effect for alphabets.

- Display numbers (0–9) instead of alphabets.

- Arduino sketch that reproduces this exact style on an 8×8 LED dot-matrix using MAX7219, with walking animation (moving legs & arms, Pedestrian Walking Man (Traffic Light Style)).

- Interface the DHT22 Temperature & Humidity Sensor with Arduino & display the sensor value on 8×32 Dot Matrix LED Display.

- With an 8×8 LED dot-matrix, displaying Dotted Heart

# Experiment Fifteen: Interfacing RFID with Arduino to Control LEDs

## 15.1 Objective

To read RFID card UID using Arduino and use it to turn ON/OFF LEDs.

## 15.2 Components Required

| Component | Quantity |
| --- | --- |
| Arduino Uno | 1 |
| RC522 RFID Reader | 1 |
| RFID Card/Tag | 1 |
| LEDs | 2 |
| 220Ω Resistors | 2 |
| Jumper wires | As needed |

## 15.3 Circuit Diagram Description

- Connect RFID to Arduino using SPI: SDA → D10, SCK → D13, MOSI → D11, MISO → D12, RST → D9 (for arduino Uno)

- For other types of arduino boards, refer to the following table showing typical pin layout used.

- Connect LEDs to D6 and D7

https://arduinointro.com/articles/projects/how-to-use-rfid-rc522-with-arduino-a-complete-beginners-guide

## 15.4 Arduino Code

#include <SPI.h>

#include <MFRC522.h>

#define SS_PIN 10

#define RST_PIN 9

MFRC522 rfid(SS_PIN, RST_PIN);

byte validCard[4] = {0xDE, 0xAD, 0xBE, 0xEF}; // Replace with your tag UID

int redLED = 6;

int greenLED = 7;

void setup() {

Serial.begin(9600);

SPI.begin();

rfid.PCD_Init();

pinMode(redLED, OUTPUT);

pinMode(greenLED, OUTPUT);

}

void loop() {

if(!rfid.PICC_IsNewCardPresent() || !rfid.PICC_ReadCardSerial()) return;

boolean match = true;

for (int i = 0; i < 4; i++) {

if(rfid.uid.uidByte[i] != validCard[i]) {

match = false; break;

}

}

if(match) {

digitalWrite(greenLED, HIGH);

digitalWrite(redLED, LOW);

} else {

digitalWrite(redLED, HIGH);

digitalWrite(greenLED, LOW);

}

delay(2000);

digitalWrite(redLED, LOW);

digitalWrite(greenLED, LOW);

rfid.PICC_HaltA();

}

7. Procedure

- Connect RFID and LEDs to Arduino.

- Upload code and open Serial Monitor.

- Tap RFID tag. LEDs turn ON/OFF based on UID.

## 15.5 Observations

| Card UID Match | LED ON |
| --- | --- |
| Yes | Green |
| No | Red |

## 15.6 Review Questions

- What is UID in RFID?

- What protocol does RFID use?

- Can RFID be used for attendance systems?

- What’s the range of RC522?

## 15.7 Conclusion

RFID is a secure way to control access and trigger outputs like LED indicators.

## 15.8 Practical exercises

1. Conduct a project with arduino and RFID to turn green ON LED if the proper card is used and print in LCD “ACCESS APPROVED” and if the card is wrong, buzzer will be ON and LCD displays “ACCESS DENIED”.

2. Project Scenario: Arduino-Based RFID Access and Power Management System for Library Automation at RP-Gishari

An Arduino-based RFID system can be applied in a smart access control system for a college campus. In this scenario, RFID tags are issued to students, faculty, and staff as part of their ID cards. At entry points to restricted areas such as laboratories, libraries, or administrative offices, RFID readers connected to Arduino boards are installed. When a user scans their RFID tag near the reader, the Arduino processes the tag’s unique ID and checks it against a preloaded database. If the tag is authorized, the system triggers a servo motor to unlock the door, logs the entry time, and displays a welcome message on an LCD screen. Unauthorized attempts are logged for security monitoring, and access is denied. This setup ensures efficient, automated, and secure access management while providing real-time monitoring capabilities.

As an Electrical and Electronics Engineering student, you are tasked with designing and implementing an Arduino-based RFID system aimed at automating access control and power conservation in the RP-Gishari College Library. The system will serve self-studying students and ensure efficient use of electrical energy. The main objective is to enhance security and energy efficiency during students' independent study sessions.

The system requirements include:

- RFID Authentication: Only authorized students with valid RFID cards can access the library.

- Automated Door Control: Upon scanning a valid RFID card, the door opens and remains open for 10 seconds, after which it automatically closes.

- Lighting Control: The library lamps should automatically turn on when access is granted, providing light only when the space is occupied.

- Invalid Access Alert: If an unauthorized card is used:

  - A buzzer should sound.

  - A Red LED should turn on as a visual warning.

# Experiment Sixteen: Interfacing GSM Module to Control LEDs via SMS

## 16.1 Objective

To send SMS commands from a phone to control LEDs connected to Arduino via a GSM module.

## 16.2 Components Required

| Component | Quantity |
| --- | --- |
| Arduino Uno | 1 |
| GSM SIM800L | 1 |
| SIM Card | 1 |
| LEDs | 2 |
| Resistors 220Ω | 2 |
| Level shifter or voltage divider | Optional |
| Jumper wires | As needed |

## 16.3 Circuit Diagram Description

- Connect GSM TX to Arduino RX (D2), GSM RX to Arduino TX (D3) via software serial

- Power GSM with external supply (3.7–4.2V)

- LEDs to D10 and D11

https://miliohm.com/sim800l-arduino-tutorial/

## 16.3 Arduino Code

#include <SoftwareSerial.h>

SoftwareSerial gsm(2, 3);

String text = " ";

int led1 = 10;

int led2 = 11;

void setup() {

gsm.begin(9600);

Serial.begin(9600);

pinMode(led1, OUTPUT);

pinMode(led2, OUTPUT);

}

void loop() {

if(gsm.available()) {

text = gsm.readString();

Serial.println(text);

if(text.indexOf("ON1") >= 0){

digitalWrite(led1, HIGH);}

if(text.indexOf("OFF1") >= 0) {

digitalWrite(led1, LOW);}

if(text.indexOf("ON2") >= 0){

digitalWrite(led2, HIGH);}

if(text.indexOf("OFF2") >= 0) {

digitalWrite(led2, LOW);}

}}

## 16.4 Procedure

- Insert SIM card and power GSM.

- Upload code to Arduino.

- Send SMS with content “ON1” or “OFF1” to control LED1, and so on.

## 16.5 Observations

| SMS Text | LED State |
| --- | --- |
| ON1 | LED1 ON |
| OFF1 | LED1 OFF |
| ON2 | LED2 ON |
| OFF2 | LED2 OFF |

## 16.6 Review Questions

- What is the function of a GSM module?

- Why do we use SoftwareSerial?

- Can GSM modules receive voice?

- How to secure GSM-based systems?

## 16.7 Conclusion

This experiment demonstrates remote control of devices using GSM, useful for rural automation and security systems.

“Success is no accident. It is hard work, perseverance, learning, studying, sacrifice and most of all, love of what you are doing or learning to do.” ― Late Pelé, Brazilian pro footballer.

………………………. The end…………………